Non-adjunct EMU Bus Hardware/Software

Interface Control Document

Motorola, Inc.

Personal Communications Sector
Razor Product Group

October 24, 2003

1.	Revision History	6

2.	Introduction	7

2.1.	Purpose and Scope	7

2.2.	Nomenclature and Conventions	7

2.2.1.	Acronyms and Abbreviations	7

2.3.	Contact Information	7

2.4.	References	7

3.	Hardware Interface	8

3.1.	EMU Block Diagram	8

3.2.	Detailed Signal Description	8

3.2.1.	Power	9

3.2.2.	Communication (USB/RS232)	10

3.2.3.	Interrupt and Control	10

3.2.4.	Audio	11

3.3.	GPIO Usage	12

3.3.1.	Statically Configured GPIO	12

3.3.2.	Dynamically Defined GPIO	12

4.	Software Interface	12

4.1.	Neptune Configuration	12

4.1.1.	GPIO Configuration	12

4.1.2.	External Interrupt Configuration	13

4.2.	PCAP Configuration	13

4.2.1.	Interrupts	13

4.2.1.2.	USB_4_VI	14

4.2.2.	General Control	14

4.2.3.	A/Ds	14

4.3.	Detection and Identification	15

4.3.1.	Detection	15

4.3.2.	Identification	16

4.4.	Device Handling	18

4.4.1.	USB CABLE:	18

4.4.2.	Factory Mode:	19

4.4.3.	SW Regression Mode:	19

4.4.4.	Smart SPD or PPD:	19

4.4.5.	Chargers (MPx and EMU):	19

4.4.6.	EMU SIHFs:	19

4.4.7.	Mono EMU Headset	20

4.5.	Charging and Metering	20

4.5.1.	Hardware Control of Power Paths	20

4.5.2.	USB Host Charging	20

4.5.3.	Midrate Charging	21

4.5.4.	Fullrate Charging	22

4.5.5.	Battery Metering	22

4.5.6.	Charging Flowcharts	22

4.6.	Smart Device Support	28

4.6.1.	Smart Device Identification	28

4.6.2.	Audio/UART Mode Switching	30

5.	Factory Interface	31

5.1.	Test Command Requirements	31

5.1.1.	EMU_AUDIO (Official Name TBD) Test Command	31

5.1.2.	SET_CHARGER Test Command	32

5.2.	Test Coverage	32

5.2.1.	Test Bay Requirements	32

5.2.2.	Testing Details	32

6.	Philips ISP1109 Addendum	35

6.1.	Hardware Signals	35

6.2.	ISP1109 SPI Interface Specification	36

6.2.1.	ISP1109 SPI Transfer Settings and Data Format	36

6.2.2.	ISP1109 SPI Register Map	36

6.2.3.	ISP1109 Register Definitions	37

6.3.	Statically Configured GPIO	39

6.4.	Dynamically Configured GPIO	39

6.5.	Interrupt Handling	39

6.6.	Bus Configuration	40

6.7.	Factory Mode Detection and Factory Mode	40

6.8.	External Power Path Control	40

6.9.	Factory Considerations	40

6.9.1.	Test Command Requirements	40

6.9.2.	Turn On	40

6.9.3.	EMU_AUDIO Test Command Changes	40

6.9.4.	ID Line Test coverage	41

Figures

Figure 3-1: Razor EMU Bus Block Diagram	8

Figure 4-1: PPD_INT_B Debounce	16

Figure 4-2: Self Powered Device Identification	17

Figure 4-3: Phone Powered Device Identification	18

Figure 4-4: Radio Off, No Battery or Vbus device, Attach Vbus device	22

Figure 4-5: Radio Off, Battery Present, Attach Vbus Device	23

Figure 4-6: Charge_USB_RX	24

Figure 4-7: Charge_All_RX	25

Figure 4-8: Charge_Fast_TX	26

Figure 4-9: Charge_Mid_TX	27

Figure 4-10: Charging USB TX	28

Figure 4-11: Smart Device Identification	29

Figure 4-12: Phone Initiated Audio to UART Mode Switch Ladder	30

Figure 4-13: Accessory Initiated Audio to UART Mode Switch Ladder	30

Figure 5-1: EMU_AUDIO Test Command Flowchart	31

Figure 5-2: Factory Sequence for Radio Turn-on	33

Figure 5-3: ID Line Testing Flowchart	34

Figure 6-1: SPI Transfer Format	36

Figure 6-2: EMU_AUDIO Test Command (Radio Perspective)	41

Figure 6-3: ID Line Test Coverage for ISP1109 Based Systems	41

Tables

Table 3-1: Consolidated Signal Description	9

Table 3-2: EMU Interrupt Sources	10

Table 3-3: EMU Control Signals	10

Table 3-4: Bus Mode Control	11

Table 3-5: Statically Defined GPIO	12

Table 3-6: Dynamically Configured GPIO	12

Table 4-1: GPIO Configuration Reference	13

Table 4-2: PCAP Interrupts, Sense, and Masks	14

Table 4-3: PCAP General Control Signals	14

Table 4-4: A to D Thresholds for Device Identification	15

Table 4-5: Default Signal States for Detection and Identification	15

Table 6-1: ISP1109 Based EMU Signals	35

Table 6-2: ISP1109 Register Map	37

Table 6-3: MCR1 Bit Definitions	37

Table 6-4: MCR2 Bit Definitions	37

Table 6-5: ACR Bit Definitions	38

Table 6-6: TCR Bit Definitions	38

Table 6-7: RCR Bit Definitions	38

Table 6-8: ISR Bit Definitions	38

Table 6-9: ILR Bit Definitions	38

Table 6-10: IEN_LOW Bit Definitions	39

Table 6-11: IEN_HIGH Bit Definitions	39

Table 6-12: Statically Configured GPIO	39

Table 6-13: Bus Configuration Settings	40

# Revision History

| Revision | Date | Author(s) | Reason |
| --- | --- | --- | --- |
| 0.1 | 10/1/2003 | Don La Monica Tim McCune | Initial Draft. Incomplete Release. |
| 0.2 | 10/24/2003 | Don La Monica Tim McCune | Initial Release for review. Major additions/editing. Smart device section incomplete (not needed for phase 1 or Razor SA). |
| 0.3 | 10/27/2003 | Tim McCune Don Lamonica | Corrected SW_BP_EN to PA6 in tables 3-5 and 4-1 Corrected PPD_DET_I to PPD_DET_B in section 4.3.1.2 General corrections (spelling/grammar) “Apps” added in figure 4-6 Corrected USB_PS description Swapped Mono Headset and not used in Table 3-4 Changed 0 and 1 to ASSERTED and DEASSERTED for clarity in table 3-4 Corrected HAPI_USB_HW and HAPI_USB_HW_5 in table 4-1 Changes related to dual path charging PA13 Changed to MID_RATE_CTRL SW_CUR_SEL removed Changes to device handling for USB cable PE12 Changed to FACT_DET (High voltage ID detect) Added content to Factory Interface Section Charging Sections changed to reflect modified dual path architecture. |
| 0.4 | 12/16/03 | Tim McCune Don La Monica | PPD identification flow chart changed to reflect changes in the EMU specification. Changed MUX lines to original configuration (due to leakage paths) Added current check prior to turn on. Clarification on stereo tests (only SPKR_R needs to be tested). Added FACT_DET control of external power path Removed HV_FLASH Added HAPI signal cross-reference for all hardware signals Added ISP1109 (Philips EMU IC) Addendum Added Charge_USB_RX flow chart Updated Charge_MID_TX |
| 0.5 | 3/17/2004 | Tim McCune | Updated title to reflect expanded scope. Updated software regression and factory modes wrt charging. Updated SPD detection for SHIF Added SET_CHARGER test command |

# Introduction

## Purpose and Scope

This document is meant as a design guide for the hardware and software implementation of the EMU bus based on the use discrete components or Philips ISP1109 IC, Neptune LTE, and PCAP2.  The upper layers of EMU bus support (anything above rtime) should not change when moving to a fully integrated solution; however some degree of low level change is expected.

Once completed all aspects of the HW/SW interface for this EMU bus implementation will be covered within this document

## Nomenclature and Conventions

### Acronyms and Abbreviations

USB	Universal Serial Bus

EMU	Enhanced Mini USB

PPD	Phone Powered Device

SPD	Self Powered Device

SE1	Singled Ended 1

ISR	Interrupt Service Routine

## Contact Information

Any document issues, questions, or input should be relayed to the following:

Tim McCune		Tim.McCune@motorola.com		+1-847-523-2735

Don La Monica		D.Lamonica@motorola.com		+1-847-523-8285

## References

All EMU bus specifications can be found at:

compass.mot.com/go/emu

EMU Bus: Audio Architecture

EMU Bus: Power Architecture

EMU Bus: Detection, Identification, and Control

# Hardware Interface

## EMU Block Diagram

Figure 3-1: Razor EMU Bus Block Diagram

## Detailed Signal Description

| Signal | Connection | Description |
| --- | --- | --- |
| VBUS | Mini USBEMU control logic & power | Supplies power to the radio from SPD’s. Acts as SW_B+ for PPD’s. Used in SPD detection |
| D+ | Mini USBEMU control logic & muxing | Used for device identification. Acts as D+ for USB mode, UART RXD in UART mode MIC_IN in audio mode SPKR_R in stereo mode |
| D- | Mini USB EMU control logic & muxing | Used for device identification. Acts as D- for USB mode, UART TXD in UART mode SPKR_OUT in audio mode SPKR_L in stereo mode |
| ID | Mini USB  EMU control logic & sense | Used for PPD detection, Device identification. Acts as MUTE to SIHF (controlled by SNP_INT_CTL) Acts as SEND/END for headset |
| MID_RATE_CTRL | Neptune  EMU charging control logic | Controls the external power supply path |
| SNP_INT_B | EMU Detection hardware  Neptune | Signal Negotiation Protocol Interrupt |
| CHRG_DET_PU_B | Neptune EMU control logic |  |
| SNP_INT_CTL | EMU Detection hardware  Neptune | Control signal to pull the ID line low. |
| USB_EN_B | Neptune  EMU Power control |  |
| MUX1 | Neptune  Audio/Data selection mux | Function multiplexor selection line |
| MUX2 | Neptune  Audio/Data selection mux | Function multiplexor selection line |
| SW_BP_EN | Neptune  EMU Power control | Switched B+ enable |
| PPD_INT_B | EMU Detection hardware  Neptune | Phone Powered Device Detect |
| SPKR_R | PCAP Audio/Data selection mux | Speaker right input to audio/data mux |
| SPKR_L | PCAP Audio/Data selection mux | Speaker left input to audio/data mux. Also used for SPKR_OUT for headset/car kit |
| MIC_IN | Audio/Data selection mux  PCAP | MIC input to PCAP from audio/data selection mux |
| FACT_DET | EMU Control logic  Neptune | Indicates factory mode entered by elevated ID voltage at power up. Used to enable the external power path under SW control |
| EMU_2.8 | 2.8V reg  EMU logic |  |
| EMU_3.3 | 3.3V reg  EMU logic |  |
| AD6 | ID PCAP | Used to sense the voltage on the ID line |
| USB_PWR | EMU control  PCAP | Gated version og VBUS to PCAP USB power input. |
| USB_TXENB | Neptune  PCAP | Used in USB mode to enable USB TX. Controlled from USB module |
| USB_VPIN | PCAP  Neptune | Used for VPIN in USB mode. Used to sense the D+ state for PPD identification. |
| USB_XRXD | PCAP  Neptune | USB receive data for USB mode |
| USB_SE0 | Neptune  PCAP | Single ended 0 generation in USB mode |
| USB_VMIN | PCAP  Neptune | VMIN during USB mode. Used to sense the D- state during device identification. Used as UART RXD in UARTmode |
| USB_VPOUT | Neptune  PCAP | USB TX data in USB mode. UART TXD in UART mode |

Table 3-1: Consolidated Signal Description

### Power

The EMU bus allows charge current to be supplied by the VBUS pin. Supported VBUS sources will be Motorola Chargers, CEA-936 compliant Car-kits, USB hosts, or a factory mode supply.  The charger path will be a dual-path topology with a hardware controlled discharge lockout when connected to a USB host prior to software charge current negotiation.

A Fast-Rate Charger (>1Amp capability) and a Mid-Rate Charger (>450mA capability) will be standard Motorola EMU accessories.  Razor will charge from either of these charger types once they are identified as being valid.  Validity will be based on a valid USB_ID value as well as a valid voltage range (4.7V-5.25V).  Both will be treated as a Standard Charger in RX due to power dissipation constraints.  If charger is invalid, software will not charge.  EMU Chargers are not compatible with P2k or LCA.

A Factory Mode can be entered by applying VBUS voltage to the USB_ID pin.  The purpose of Factory Mode is to allow power-up to occur without a battery.

When VBUS is supplied by a USB host, hardware detection will default charger to off until SW powers up and negotiates 500mA with host. If 500mA is negotiated, software will begin to charge battery.  If 500mA is denied, then charging will not occur and the USB host will be treated as a data cable only.

The Radio will also have the capability to supply a switched B+ supply to VBUS originating from the battery.  This supply will allow phone powered accessories to receive power from the phone.  This supply will be controlled by SW_BP_EN.

### Communication (USB/RS232)

Along with the standard USB communication between a phone and a host, another UART based protocol will also be supported.  This protocol will allow a phone to communicate with a CEA-936 compliant device without the expense of requiring it to be a USB Host.

Standard USB communication will occur by utilizing the base band USB controller and UART.  USB and UART muxing will occur in PCAP2.

Software will need to put PCAP in the appropriate mode by driving USB_EN_B.

### Interrupt and Control

The following Interrupts will be generated to indicate changes in EMU Bus state.

| Hardware Source | Logical Interrupts | Function |
| --- | --- | --- |
| PCAP_INT | USB4VI | Indicates when VBUS added or removed. Currently proposed not to be used. All SPD insertion and removal shall be performed using MOBPORTBI |
|  | MOBPORTBI | indicates when charger is added or removed |
| SNP_INT_B | NA | Indicates accessory is initiating communication with radio |
| PPD_INT_B | NA | Allows radio to initiate communication with accessory |

Table 3-2: EMU Interrupt Sources

The following signals will be used for EMU Bus Control

| Signal | Function |
| --- | --- |
| MUX1 | Switches appropriate signals on D+ / D- of min-USB connector |
| MUX2 | Switches appropriate signals on D+ / D- of min-USB connector |
| SNP_INT_CTRL | interrupts accessory to request it to enter UART mode |
| FACT_DET | Detects elevated ID voltage factory mode. Also used to enable the external power path via software. |
| MID_RATE_CTRL | Controls the external power path connection |
| CHRG_DET_PU | Connects a pull-up resistor on D+ when radio is not in USB mode |
| USB_EN_B | Used to control the VBUS pass device to allow PCAP to detect the voltage on VBUS and switch to USB mode. |
| SW_BP_EN | Enables the supply to phone powered devices. Also places phone powered devices in low power mode. |

Table 3-3: EMU Control Signals

#### Device to Device Communication Usage

The following signals are required to communicate with accessories:

PPD_INT_B: A falling edge indicates that a phone powered device has been inserted.  This interrupt should be masked when a self powered device is detected.  This interrupt will be asserted in conjunction with the SNP_INT_B for a self powered device due to the nature of the hardware.

SNP_INT_B:  A falling edge indicates a smart device request to enter UART mode and begin communication.  This interrupt should be masked during a MS Accessory SNP_INT_B initiated by asserting SNP_INT_CTL.  This signal also serves the SEND/END functionality for the EMU headset.  There will be different ISR’s registered for this interrupt based on bus mode.

SNP_INT_CTL:  Software will control the bus mode (audio or UART) to CEA-936 compliant accessories by driving SNP_INT_CTL as necessary (see x.x.x).  This hardware signal is also used for MUTE control of the SIHF.  In this mode its polarity is reversed (H = SIHF un-muted, L= SIHF muted).  For maintainability it may be useful to define a separate HAPI signal the SIHF MUTE functionality.

MUX1, MUX2, USB_EN_B: The truth table below indicates how MUX1,  MUX2 and USB_EN are used to place the bus in the appropriate state to switch in the correct signals.

| Mode | MUX1 | MUX2 | USB_EN_B |
| --- | --- | --- | --- |
| USB mode | 0 | 0 | ASSERTED |
| UART mode | 0 | 0 | DEASSERTED |
| Not used | 0 | 1 | X |
| Mono headset / carkit | 1 | 0 | X |
| Stereo mode | 1 | 1 | X |

Table 3-4: Bus Mode Control

SW_BP_EN: See Table 3-3

#### Internal Control Usage

The following signals are used to control devices internal to the radio to allow charging, detection and for some mode changes:

MID_RATE_CTRL: See Table 3-3

FACT_DET: This signal is used to detect fatory mode at power up.  After check initial states this signal acts as the software control signal to enable the external power path.  It should be set low when there is no external power present to conserve power.  FACT_DET should be driven high whenever software must ensure the external power path remains connected (e.g. during Charger/SIHF identification).

CHRG_DET_PU: See Table 3-3

### Audio

Audio and data share the same pins on the mini-USB connector (D+ and D-).  Supported Audio accessories will be a mono-headset with send/end, a car-kit (mono audio and mic) and possibly a stereo headset.  The audio interface will meet the CEA-936 requirements.  Note that Razor will not have a separate headset-jack due to space constraints and thus will have a mini-USB based headset.  The gains will be same for a car-kit and headset (with amplifiers in the headset to change gain as needed).  Echo cancellation will need to be disabled when headset audio is being sent.

## GPIO Usage

### Statically Configured GPIO

These signals retain the same GPIO configuration regardless of the operating mode of the bus.  This section serves as a quick reference for the GPIO connectivity; signal definitions and usage are covered in other areas of this specification.

| GPIO Pin | Signal(s) | Neptune Module | Reuse from Triplets |
| --- | --- | --- | --- |
| PA6 | SW_BP_EN | MCU GPIO | Y |
| PA11 | CHRG_DET_PU | MCU GPIO | N |
| PA12 | PPD_INT_B | EXT INT 3 | N |
| PA13 | MID_RATE_CTRL | MCU GPIO | (Y) |
| PD8 | USB_EN_B | MCU GPIO | N |
| PD10 | USB_TXENB | USB | Y |
| PD11 | USB_VPIN | USB | Y |
| PD13 | USB_XRXD | USB | Y (no RTS) |
| PD15 | USB_SE0 | USB | Y |
| PE1 | SNP_INT_CTL | MCU GPIO | N |
| PE3 | SNP_INT_B | EXT INT 4 | N |
| PE10 | MUX1 | MCU GPIO | N |
| PE11 | MUX2 | MCU GPIO | N |
| PE12 | FACT_DET | MCU GPIO | N |

Table 3-5: Statically Defined GPIO

### Dynamically Defined GPIO

These GPIO are used for different signals depending on the bus state.

| GPIO Pin | Signal |  | Neptune Module |  | Reuse from Triplets | Cross Reference |
| --- | --- | --- | --- | --- | --- | --- |
|  | USB Mode | UART Mode | USB | UART |  |  |
| PD12 | USB_VMIN | URXD1 | USB | UART1 | Y |  |
| PD14 | USB_VPOUT | UTXD1 | USB | UART1 | Y |  |

Table 3-6: Dynamically Configured GPIO

# Software Interface

## Neptune Configuration

### GPIO Configuration

Table 4-7 contains the information required for the GPIO configuration.  The required defines used by HAPI can be generated by placing “HAPI_GPIO_” prior to the hardware signal, port, and data direction columns.  Those signals that have HAPI signals defined in this table already have all the required defines in hapi_gpio_defs.h, and hapi_neptune_portlist.h.  They also have the required table entries in place in hapi_neptune_portlist.c.  Entries with HAPI signals listed in () have the equivalent physical configuration as the listed HAPI signal, but different logical usage.

GPIO initialization states are indicated in the default column.  If a pin has multiple uses the GPIO selection muxes and direction should reflect the setting for the signal that has an entry in the default column.  The default state (ACTIVE/INACTIVE) of signals with an * in the default column is not configurable.  For those signals with a default state indicated the corresponding bit in MCU data register for the corresponding port should be set to this state.

| Hardware Signal | Port | Pin | Data Direction | Output Selection | Input Selection | Default | Active State | HAPI signal |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SW_BP_EN | PORT_A | 6 | OUTPUT | 0 | - | INACTIVE | HIGH | HAPI_SW_BPLUS_EN |
| CHRG_DET_PU_B | PORT_A | 11 | OUTPUT | 0 | - | ACTIVE | LOW | HAPI_CHRG_DET_PU_B |
| PPD_INT_B | PORT_A | 12 | INPUT | - | 0 | * | LOW | HAPI_PPD_INT_B |
| MID_RATE_CTRL | PORT_A | 13 | OUTPUT | 0 | - | INACTIVE | HIGH | HAPI_SW_CUR_SEL |
| USB_EN_B | PORT_D | 8 | OUTPUT | 0 | - | ACTIVE | LOW | HAPI_USB_EN_B |
| USB_TXEN_B | PORT_D | 10 | OUTPUT | 2 | - | * | LOW | HAPI_USB_HW_2 |
| USB_VPIN | PORT_D | 11 | INPUT | - | 0 | * | NA | HAPI_USB_HW_6 |
| USB_VMIN | PORT_D | 12 | INPUT | - | 0 | * | NA | HAPI_USB_HW_5 |
| URXD1 | PORT_D | 12 | INPUT | - | 2 |  | NA | HAPI_UART1_RX1_DATA |
| USB_XRXD | PORT_D | 13 | INPUT | - | 0 | * | NA | HAPI_USB_HW_3 |
| USB_VPOUT | PORT_D | 14 | OUTPUT | 2 | - | * | NA | HAPI_USB_HW_4 |
| UTXD1 | PORT_D | 14 | OUTPUT | 5 | - |  | NA | HAPI_UART1_TX1_DATA |
| USB_SE0 | PORT_D | 15 | OUTPUT | 2 | - | * | HIGH | HAPI_USB_HW |
| SNP_INT_CTL | PORT_E | 1 | OUTPUT | 0 | - | INACTIVE | HIGH | HAPI_SNP_INT_CTL |
| SNP_INT_B | PORT_E | 3 | INPUT | - | 1 | * | LOW | HAPI_SNP_INT_B |
| MUX1 | PORT_E | 10 | OUTPUT | 0 | - | ACTIVE | HIGH | HAPI_MUX_1 |
| MUX2 | PORT_E | 11 | OUTPUT | 0 | - | ACTIVE | HIGH | HAPI_MUX_2 |
| FACT_DET | PORT_E | 12 | INPUT | 0 | 0 | * | HIGH | HAPI_FACT_DET |

Table 4-7: GPIO Configuration Reference

### External Interrupt Configuration

There are only three physical external interrupt sources used for EMU bus.  These are the PCAP interrupt connected to INT1 and configured exactly as it is in Triplets (no code changes required); the PPD_INT_B connected to INT2 and configured exactly as the Option select 2 interrupt in Triplets (probably should create a new signal for clarity, but can copy defines); and the SNP_INT_B which must be reconfigured based on the attached accessory.

## PCAP Configuration

### Interrupts

PCAP interrupts will be indicated to software by the PCAP_INT signal.  Software will then read the ISR and PSTAT registers to determine source of interrupt and debounce as required.

The following PCAP interrupts will be used:

| Interrupt | PCAP Register | Bit | Bit Definition |
| --- | --- | --- | --- |
| MOBPORTBI | Reg 0: ISR | 10 | 1=interrupt generated, write 1 to clear |
| MOBPORTBM | Reg 1: IMR | 10 | 1=MobportBI is masked |
| MOBPORTBS | Reg 2:PSTAT | 10 | 0=MobportB not present, 1=MobportB present |
| USB4VI | Reg 0:ISR | 6 |  |
| USB4M | Reg 1:IMR | 6 |  |
| USB4S | Reg 2:PSTAT | 6 |  |

Table 4-8: PCAP Interrupts, Sense, and Masks

#### MOBPORTI

This interrupt will be used to indicate when any self powered device is attached.  It is also used to detect the removal of any SPD (including the USB data cable).  An interrupt will be generated on a rising and falling edge of MobportB.  MobportI has corresponding sense and mask bit

### USB_4_VI

Currently this interrupt is unused.  Only the USB_4_VS bit is used during identification of the attached accessory.

### General Control

The following signals are used for general control and are accessed through PCAP control registers.

| Hardware Signal | PCAP Register | Bit | SW Init State | Active State | HAPI signal |
| --- | --- | --- | --- | --- | --- |
| USB_PU | 0x14 | 2 | DE-ASSERTED | HIGH | PCAP1_HAPI_USB_VCCRENB |
| VUSB_EN | 0x14 | 4 | DE-ASSERTED | HIGH | HAPI_VUSB_EN |
| USB_PS | 0x14 | 5 | ASSERTED | HIGH | HAPI_USB_PS |
| RS232ENB | 0x14 | 9 | DE-ASSERTED | LOW | PCAP1_HAPI_RS232_TRANSCEIVER_EN |
| RS_232_DIR | 0x14 | 10 | ASSERTED | HIGH | HAPI_RS_232_DIR |

Table 4-9: PCAP General Control Signals

USB_PU: Used to enable the 1.5k pull up on the D+ line.  Asserted to begin the enumeration process once a USB host is detected.

VUSB_EN:  Manually enables the USB transceiver.  The USB transceiver must be manually enabled to properly identify PPD’s and during factory mode.  Since all currently defined SPD’s will supply > 4V on VBUS the USB transceiver should be enabled simply by asserting USB_EN_B during SPD identification.

USB_PS:  This signal is used to select the supply input for the PCAP internal USB regulator.  USB_PS should be de-asserted only during PPD device identification and factory mode.

RS232ENB:  Enables the RS-232 transceiver when asserted.  This signal is overridden if USB power is sensed by PCAP.  RS232ENB must be asserted during UART communication modes.  It must be de-asserted during audio mode.

RS_232_DIR:  Controls the lines to which UART TXD and RXD are connected.  Currently this signal should be asserted at all times.

### A/Ds

PCAP A/D’s will be used for device identification as well as battery and charge metering.

#### A/D Thresholds

The table below shows the A/D thresholds for PCAP channel AD6 which will be used for device identification.  Since a resistor value depicts different accessories depending on context, only the resistor value is shown.

| ID Resistor | Counts Min | Counts Max |
| --- | --- | --- |
| Open | 215 | 255 |
| 440k | 157 | 214 |
| 200k | 113 | 156 |
| 102k | 52 | 112 |
| 10k | 10 | 51 |
| 1k | 0 | 9 |

Table 4-10: A to D Thresholds for Device Identification

## Detection and Identification

This section covers the details of configuration and low-level sequencing required for detection and identification of EMU bus accessories.  The high level design is available in the EMU Bus Detection and Identification Specification.

### Detection

Device detection is accomplished through 2 sources.  Self powered device attachment is detected through the MOBPORTBI generated by PCAP2.  Phone powered device attachment and removal is detected through the PPD_INT_B signal.

#### Default Signal States for Detection and Identification

The table below provides the required default states of the control signals to enable proper accessory detection.

| Signal | Idle (no accessory) State | Source | Notes |
| --- | --- | --- | --- |
| USB_EN_B | ASSERTED | Neptune GPIO | Needs to be asserted to allow USB transceiver to be enabled when an SPD is detected for polling D- |
| CHRG_DET_PU_B | ASSERTED | Neptune GPIO | Connects 200k PU to D+ for charger/smart device identification |
| SNP_INT_CTL | DE-ASSERTED | Neptune GPIO | Must be de-asserted to allow proper detection/identification of the ID line |
| USB_PU | DE-ASSERTED | PCAP | Must be de-asserted to avoid being detected by a USB host during detection/identification. Should be switched in by USB driver (no different that CE bus) |
| MUX1 | ASSERTED | Neptune GPIO | Must be asserted to ensure no load ing from the audio lines. |
| MUX2 | ASSERTED | Neptune GPIO | Must be asserted to ensure no load ing from the audio lines. |
| RS232_EN_B | DE-ASSERTED | PCAP | Must be de-asserted to ensure RS-232 transceiver dos not drive TXD line |
| MOBPORTBM | DE-ASSERTED | PCAP | De-asserted to ensure MOBPORTBI is generated for SPD detection. |

Table 4-11: Default Signal States for Detection and Identification

#### Debounce and Race condition handling

When a self powered device is detected (PCAP interrupt received with the MOBPORTBI bit set) the software should clear the MOBPORTB interrupt and debounce the insertion for TBD ms (current CE bus accessory debounce is 8 x 120ms).  The debounce can be accomplished by polling the MOBPORTBS and MOBPORTBI bits in PCAP2.  During the debounce of SPD detection both the MOBPORTBI and the PPD_DET_B interrupts should be masked.

Due to the MPx charger there will be cases in which both the PPD_DET_B and MOBPORTBI/S signals will be active.  This situation also has an inherent race condition between the tow of these interrupts.  It is proposed this race condition be handled within the debounce of the PPD_DET_B interrupt.  Below is an example of how to handle the debounce.

Figure 4-2: PPD_INT_B Debounce

### Identification

Device class identification is performed by rtime using a combination of the D+ and D- lines in conjunction with the voltage level on the ID pin (read by AD6).  There are four classes of SPD’s and three classes of PPD’s.

SPD device classes are:

- USB Host devices which include

  - Standard USB Cable

  - Factory Mode USB

  - Software regression USB

- Chargers including

  - Fast charger

  - Mid rate charger

  - MPx Chargers

- Dumb self powered audio devices (Fast and mid rate SIHF)

- Smart self powered UART device

  - Smart Audio devices

  - Other smart devices

All SPD device classes are identifiable by rtime.  Devices within classes 1 through 3 can be fully identified within rtime.  Upper layer interaction (connectivity, apps) is required to fully identify any class 4 device.

PPD device classes are:

- USB OTG devices (outside the scope of this document)

- Dumb PPD’s (Headset, Quill)

- Smart phone powered devices

  - Smart phone powered audio device (eg FM Radio)

  - Other smart phone powered devices

The following sections detail the specifics of device identification for SPD’s and PPD’s.

#### Self Powered Device Identification

Figure 4-3 diagrams the sequence that should be used for the identification of the various SPD’s.

Figure 4-3: Self Powered Device Identification

#### Phone Powered Device Identification

Figure 4-4: Phone Powered Device Identification

## Device Handling

Following are the descriptions of the different modes/devices and the types of messages that should be generated between components.  These descriptions can be used for the design and implementation of any new messaging required.

### USB CABLE:

Once a USB cable has been identified by rtime the following events should occur:

Note: If the user attaches a charger very slowly there is a chance it will be ID’d as a USB cable.  Due to this issue rtime should continue to poll for an SE1 until the radio has enumerated.

- If the radio has powered up as a result of cable insertion rtime powers down the radio.  With the addition of dual path charging capability there will be no current path from USB_PWR to the battery when the radio is off therefore the radio can remain off when a data cable is inserted.

- rtime notifies connectivity of the USB cable attachment (same as CE bus)

- rtime notifies SBCM of the attachment of a USB_HOST_CHARGER (new)

- SBCM requests current capability through connectivity

- Connectivity notifies SBCM of host current capability

- If current capability is 500mA, SBCM treats as a midrate charger (end)

- If current capability is 100mA no charging is performed.  This is a Razor specific implementation.  Future EMU bus products may allow trickle charging when the USB host allows only 100mA.

### Factory Mode:

This mode is used as a replacement for the generic external power power up case of CE bus.  No charging is allowed in this mode.  The following events should be generated:

- If the radio has powered up as a result of the cable insertion rtime notifies DL of an external power power up.

- Assert VUSB_EN, de-assert USB_PS, mask MOBPORTBI.  Needed to allow for testing on the USB_PWR signal without disrupting USB communications in the factory.

- rtime notifies connectivity of USB cable insertion.  Connectivity should not negotiate for charging current.

- rtime notifies SBCM of external power present. (may not be needed)

### SW Regression Mode:

This mode allows the SW regression test station to power up the radio from external power while maintaining the ability to charge.  Charging shall be disabled until a set charger type test command has been received (see section xxxxxxxxxx).

- If the radio has powered up as a result of the cable insertion rtime notifies DL of an external power power up.

- rtime notifies connectivity of USB cable insertion. Connectivity should not negotiate for charging current.

### Smart SPD or PPD:

These modes are covered in Section 4.6.

### Chargers (MPx and EMU):

- If the radio has powered up as a result of device insertion rtime notifies DL of a charger power up.

- rtime sets the current limit according to the type of charger attached notifies SBCM of charger attachment.

### EMU SIHFs:

Note: The MUX1/2 lines should be set to the appropriate audio mode prior to enabling the audio amplifiers.

- If the radio has powered up as a result of device insertion rtime notifies DL of a charger power up.

- rtime notifies DL of SIHF attachment

- rtime sets the current limit according to the type of charger attached notifies SBCM of charger attachment.

### Mono EMU Headset

Note : The MUX1/2 lines should be set to the appropriate audio mode prior to enabling the audio amplifiers.

- rtime notifies DL of headset attachment.

- DL notifies audio of device?

- rtime registers the SNP_INT_B interrupt for SEND/END functionality.

## Charging and Metering

Battery charging scheme will be similar to P2k/Triplets which is a dual path configuration.

Changes to algorithms will be made to accommodate new voltage and current requirements of EMU Bus.  Charge and discharge metering will remain the same as triplets.

### Hardware Control of Power Paths

Hardware control in the phone will prevent discharging from the USB bus until software has negotiated 500mA with the host.  This control will force a power-up to occur from the battery instead of external power when connected to a host.   This means that charging from the host will not be possible if the battery is dead

This hardware control will be active before software is executing and will be overridden by software at power up by MID_RATE_CTRL.

At initial power up, charger detection must occur prior to configuring MID_RATE_CTRL.   The following will determine how to configure MID_RATE_CTRL at power up:

No Charger present: 		MID_RATE_CTRL=1

USB Host present:		MID_RATE_CTRL=1

Midrate Charger present:	MID_RATE_CTRL=0

Fullrate Charger present:	MID_RATE_CTRL=0

In addition, MID_RATE_CTRL must be driven high prior to entering TX mode when a charger is not present.  This is to prepare for a USB host or Midrate Charger to be inserted.

### USB Host Charging

#### USB Charging in RX

If the battery has enough capacity to allow a power up to occur when connected to a host, software must immediately negotiate 500mA for charging.  If 500mA is granted, then software will continue to drive MID_RATE_CTRL high and begin to charge the battery.   If 500mA is denied, then software will program the DAC to OFF (this is for future compatibility) and will continue to drive MID_RATE_CTRL high. The user will not see a charge indication and standard discharge metering will be used.

When charging in RX over USB, the standard CC/CV charging algorithm will be used but the phone will be set up in a single path configuration.  Because of this, provisions will be made to enter CC mode if voltage drops while in CV mode. Current will be ramped up in steps as is done with a Midrate charger in existing products.  The steps and current will be redefined such that:

CHARGE_RX_RANGE_1:	Batt<3.3V, 	Icharge=250mA

CHARGE_RX_RANGE_2:	3.3<Batt<3.7, 	Icharge=350mA

CHARGE_RX_RANGE_3:	3.7<Batt<4.2V, 	Icharge=450mA

#### USB Charging in TX

When TX is entered when attached to a USB host, a variant of the Midrate Current Share Mode will be used.  Previously, the DAC would be set to 500mA which would collapse the Midrate supply.  It is not allowable to collapse  the supply of a USB host.  Instead, the charge current must be set to different values based on battery voltage for thermal management.

The TX algorithm will operate such that initial charge current will be based on the following:

USB_CHARGE_TX_RANGE_1:	Batt<3.3V, 	Icharge=250mA

USB_CHARGE_TX_RANGE_2:	3.3<Batt<3.7, 	Icharge=350mA

USB_CHARGE_TX_RANGE_3:	3.7<Batt<4.0V, Icharge=450mA

Note that the above range is the same as RX ranges except that USB_CHARGE_TX_RANGE_3 has a maximum voltage of 4.0V in order to allow DAC to be enabled.

If USB_CHARGE_TX_RANGE_2 or USB_CHARGE_TX_RANGE_3 was valid, it is expected that the battery will charge up to 4.0V.  Once 4.0V is reached, the DAC is to be disabled and battery will be allowed to discharge to 3.7V.   At 3.7V, DAC will again be enabled to 450mA and DAC will be cycled within USB_CHARGE_TX_RANGE_3 continuously as is done today.

It is possible that battery will not be charged in TX and instead the radio will power off if the radio load exceeds the charge current.  As battery is discharged in this scenario, there will 75mV of hysteresis in the downward direction within the ranges after 4.0V has initially been reached.  For example, if DAC was disabled at 4.0V and battery voltage decreased to 3.7V, DAC would be set to 450mA to charge the battery.  If the battery voltage dropped to less than 3.7V, then no change would occur until the battery voltage continued to drop to 3.625V (3.625V=3.7V – 75mV).  At 3.625V then next lower range would be entered which would force DAC to be set to 350mA.  If Battery voltage rises and exceeds 3.7V, then the next range will immediately be entered and current will be set to 450mA.

### Midrate Charging

#### Midrate Charging in RX

Charging algorithm in RX will be the same for  a Midrate Charger and a Fullrate Charger.  The same CHARGE_RX_RANGE values will be used to determine current in constant current mode.

#### Midrate Charging in TX

When Charging in TX with a Midrate a variant of existing Midrate Current Share mode will be used.  The exception is that MID_RATE_CTRL will be driven high instead of existing MIDRATE_1 and MIDRATE_2 lines, and the DAC will be set to 750mA instead of 500mA.  As is done today, it must be verified that the Midrate Charger supply has collapsed by measuring voltage drop across charger path.

### Fullrate Charging

#### Fullrate Charging in RX

Charging in RX will be the same for a Midrate Charger and a Fullrate Charger.  The same CHARGE_RX_RANGE values will be used to determine current in constant current mode.

#### Fullrate Charging in TX

Charging in TX with a Fullrate charger will be the same as is done today.  The battery will not be charged and instead the charger will supply the radio current through the standard discharge path.  Upon identification of a fullrate charger, MID_RATE_CTRL must be driven low which will enable the external discharge path.

### Battery Metering

Discharge and Charge Metering will be implemented the same as is done on Triplets.

### Charging Flowcharts

Figure 4-5: Radio Off, No Battery or Vbus device, Attach Vbus device

Figure 4-6: Radio Off, Battery Present, Attach Vbus Device

Figure 4-7: Charge_USB_RX

Figure 4-8: Charge_All_RX

Figure 4-9: Charge_Fast_TX

Figure 4-10: Charge_Mid_TX

Figure 4-11: Charging USB TX

## Smart Device Support

This section details smart device operation.  Smart device operation is currently not fully defined within the CEA-936 specification

### Smart Device Identification

Smart device identification is covered in this section.  It is assumed that rtime has already identified the device as a smart device.  Refer to section 4.3.2 for device class identification.  Figure 4-12 diagrams the flow of smart device identification within the device layer.

Figure 4-12: Smart Device Identification

### Audio/UART Mode Switching

Figure 4-13: Phone Initiated Audio to UART Mode Switch Ladder

Figure 4-14: Accessory Initiated Audio to UART Mode Switch Ladder

# Factory Interface

This section covers the factory test requirements/sequencing and SW requirements for new test commands.

## Test Command Requirements

Currently there is only one new test command required for EMU bus coverage in the factory.  The majority of the testing can be completed using GPIO test commands to set and read the various GPIOs.

### EMU_AUDIO (Official Name TBD) Test Command

It is required to test the audio paths through the EMU bus connector.  This will be accomplished through the use of a new test command which will operate as follows:

- The test station shall send the EMU_AUDIO test command

- The radio will ACK the test command, then wait for 3ms (wait is negotiable)

- After the 3ms wait the radio will switch to audio mode with the EMU mic path looped back to the EMU mono speaker path.

- During audio loopback the radio shall monitor the state of the PPD_INT_B and SNP_INT_B signals.  When the PPD_INT_B is asserted the radio will disable loop back.

- If the SNP_INT_B signal is not asserted at this point the radio will switch into EMU stereo mode and generate tomes on the SPKR_L and SPKR_R lines (For factory test purposes only SPKR_R need be verified since SPKR_L was verified during loopback).  During this time the radio will continue to monitor the state of the SNP_INT_B signal.

- When the SNP_INT_B signal is asserted the radio will power down.

The flowchart below shows the sequence described above.

Figure 5-15: EMU_AUDIO Test Command Flowchart

### SET_CHARGER Test Command

In order to support testing of charging during software regression a test command must be added to set the charger type.  Upon receiving this test command SBCM should be notified of the presence of the charger type indicated in the test command data.  The charger type supported shall be:

CHARGER_TYPE_NONE

CHARGER_TYPE_USB_CHARGER

CHARGER_TYPE_MID_RATE

CHARGER_TYPE_FAST

## Test Coverage

### Test Bay Requirements

In order to effectively test EMU bus functionality the test bay must be capable of:

- Controlling the D+ and D- connections to the PC.

- Shorting D+ and D- together.

- Selectively biasing the ID pin with 4V or any resistor value from Table 4-10

- Providing an audio signal input on the D+ line (when D+ is disconnected from the PC) and monitoring the audio signal present on D- when in audio loop-back mode

- Controlling the voltage present at the USB_PWR pin.

### Testing Details

#### Radio Turn On

The following sequence will be used to turn the radio on:

Figure 5-16: Factory Sequence for Radio Turn-on

This sequence must be completed within 500ms and tests the following sub-circuits; D+/D- short detector, ID 4V detector, hardware path enable to B+, external power connectivity to the radio.

#### ID Line Test Coverage

Once the radio has enumerated the ID line must be tested for A/D values, PPD_INT_B and SNP_INT_B detection, and SNP_INT_B generation (via SNP_INT_CTL).

The A/D values may be verified by either connecting the various pull-down values or by applying the corresponding voltage to the ID line and reading AD6 through the a/D test command.  Using the various pull-down values will most likely be the more generic testing method as the actual voltage on the ID line for a given pull-down could vary from product to product.

PPD_INT_B can be tested by checking for a high on PA12 through the GPIO test command, then applying a 102k pull-down to the ID line and checking for a low on PA12.

SNP_INT_B and SNP_INT_CTRL can be tested at the same time.  First SNP_INT_B should be verified as high by reading PE3.  Next SNP_INT_CTL should be asserted by driving PE1 high.  SNP_INT_B should be read again and should be low.

Figure 5-17: ID Line Testing Flowchart

# Philips ISP1109 Addendum

This section covers the use of the Philips ISP1109 Addendum.  Every effort has been made to map the signals defined in the discrete solution to the functional equivalents in the IC.

## Hardware Signals

The hardware interface to the ISP1109 consists of various GPIOs and the base-band SPI interface.

| Signal | Connection | Description |
| --- | --- | --- |
| BB_SPI_CLK | Neptune  ISP1109, ATI, PCAP2, FL | Base-band SPI clock |
| BB_MOSI | Neptune  ISP1109, ATI, PCAP2, FL | Base-band Master Out Slave In |
| BB_MISO | ISP1109, ATI, PCAP2  Neptune | Base-band Master in Slave Out |
| ISP1109_CS (USB_EN) | Neptune  ISP1109 | ISP1109 SPI chip select. Active High |
| EMU_INT_B (PPD_INT_B) | ISP1109  Neptune | This signal replaces the individual hardware interrupts of the discrete solution. Some PCAP functionality has also been transferred to this interrupt. The actual interrupt source must be determined by reading the status register. |
| ISET_SENSE (FACT_DET) | ISP1109  Neptune | This signal is used in conjunction with the SE1 SPI bit to detect Factory Mode |
| VBUS | Mini USBISP1109, PCAP2 | Supplies power to the radio from SPD’s. Acts as SW_B+ for PPD’s. Used in SPD detection |
| D+ | Mini USBISP1109 | Used for device identification. Acts as D+ for USB mode, UART RXD in UART mode MIC_IN in audio mode SPKR_R in stereo mode |
| D- | Mini USB ISP1109 | Used for device identification. Acts as D- for USB mode, UART TXD in UART mode SPKR_OUT in audio mode SPKR_L in stereo mode |
| ID | Mini USB  ISP1109 | Used for PPD detection, Device identification. Acts as MUTE to SIHF (controlled by SNP_INT_CTL) Acts as SEND/END for headset |
| MID_RATE_CTRL | Neptune  EMU charging control logic | Controls the external power supply path |
| SW_BP_EN | Neptune  EMU Power control | Switched B+ enable |
| SPKR_R | PCAP ISP1109 | Speaker right input to audio/data mux |
| SPKR_L | PCAP ISP1109 | Speaker left input to audio/data mux. Also used for SPKR_OUT for headset/car kit |
| MIC_IN | ISP1109  PCAP | MIC input to PCAP from audio/data selection mux |
| AD6 | ID PCAP | Used to sense the voltage on the ID line |
| USB_TXENB | Neptune  ISP1109 | Used in USB mode to enable USB TX. Controlled from USB module |
| USB_VPIN | ISP1109  Neptune | Used for VPIN in USB mode. Used to sense the D+ state for PPD identification. |
| USB_XRXD | ISP1109  Neptune | USB receive data for USB mode |
| USB_SE0 | Neptune  ISP1109 | Single ended 0 generation in USB mode |
| USB_VMIN | ISP1109  Neptune | VMIN during USB mode. Used to sense the D- state during device identification. Used as UART RXD in UARTmode |
| USB_VPOUT | Neptune  ISP1109 | USB TX data in USB mode. UART TXD in UART mode |

Table 6-12: ISP1109 Based EMU Signals

## ISP1109 SPI Interface Specification

### ISP1109 SPI Transfer Settings and Data Format

Data is transferred to the ISP1109 in 32 bit words, MSB first.  The format of the data is shown in figure

SPI_CLK

Data 1

write_en

address4

address3

address2

address1

address0

Dead Bit

Data 24

Data 23

Data 0

Data 0

Data 1

Data 24

SPI_MOSI

SPI_MISO

Figure 6-18: SPI Transfer Format

The chip select for the ISP1109 SPI should be configured in the following manner:

- Chip Select Settings (CS2, CSCFG2A & B)

  - CLKDIV = 0 (13MHz SPI Clock)

  - M_L_SEL = 0 (MSB First)

  - CSPL = 0 (CS polarity High)

  - DOPH = 0 (Clock out on falling edge, device clocks in on rising)

  - DIPH  =0  (Device clocks out on falling edge, SPI clocks in on rising)

  - DAT_CNT = 1 (1 clock delay between transfers)

  - DBC_CNT = 0 (No delay before 1st transfer)

- Queue configuration (which queue used is left to the discretion of the programmer)

  - QBRST: Left to the programmer

  - QRC: Left to the programmer (QBRST has impact on this)

  - QBL = 4 (32 bit message)

### ISP1109 SPI Register Map

The following table provides the register map of the ISP1109.  Read/Write registers (RW) have separate addresses for setting and clearing bits and can be read from either of these addresses.  For RW registers the Address column contains the set address.  Read only (RO) registers have only one address.

| Register | Type | Address | Clear Address | Description |
| --- | --- | --- | --- | --- |
| Vendor ID Low | RO | 0x00 | N/A | Low byte of Philips Vendor ID |
| Vendor ID High | RO | 0x01 | N/A | High byte of Philips Vendor ID |
| Product ID Low | RO | 0x02 | N/A | Low byte of ISP1109 Product ID |
| Product ID High | RO | 0x03 | N/A | High byte of ISP1109 Product ID |
| Version ID Low | RO | 0x14 | N/A | Low byte of ISP1109 IC Version ID |
| Version ID High | RO | 0x15 | N/A | High byte of ISP1109 IC Version ID |
| MCR1 | RW | 0x04 | 0x05 | Mode Control Register 1 |
| MCR2 | RW | 0x12 | 0x13 | Mode Control Register 2 |
| ACR | RW | 0x16 | 0x17 | Audio Control Register |
| TCR | RW | 0x18 | 0x19 | Timer Control Register |
| RCR | RW | 0x06 | 0x07 | Resistor Control Register |
| ISR | RO | 0x08 | N/A | Interrupt Source Register |
| ILR | RW | 0x0A | 0c0B | Interrupt Latch Register |
| IEN_LOW | RW | 0x0C | 0x0D | Interrupt Enable Low Transition (Falling edge) |
| IEN_HIGH | RW | 0x0E | 0x0F | Interrupt Enable High Transition (Rising Edge) |

Table 6-13: ISP1109 Register Map

### ISP1109 Register Definitions

This section details the various bits/bit fields within each of the registers and their default states.  HAPI signals are given for those bits that have equivalents.

| Bit(s) | Name | Reset Value | SW Default Value | Description | HAPI Signal |
| --- | --- | --- | --- | --- | --- |
| 0 | SPEED_REG | 01 | 1 | 0=Low Speed, 1= High speed. | TDB (may not be needed since this is not going to be a dynamic setting) |
| 1 | SUSPEND_REG | 01 | 0 | 0 = Active Mode, 1 = Suspend Mode | PCAP_HAPI_USB_SUSPEND_MODE |
| 2 | DAT_SE0 | 1 | 1 | 0 = VP/VM mode, 1 = DAT-SE0 mode | TDB (may not be needed since this is not going to be a dynamic setting) |
| 3-5 | RESERVED |  | 0 | Reserved | NA |
| 6 | UART_EN | 0 | 0 | Enables the UART | !PCAP1_HAPI_RS232_TRANSCEIVER_EN |
| 7 | UART_PIN_SEL | 0 | 0 | Determines the connection of URXD and UTXD to D+ and D- | !HAPI_RS_232_DIR |

1. These bits are don’t care at power up as they are overridden by the states of the hardware pins because SPD_SUSP_CTRL is 0 by default.

Table 6-14: MCR1 Bit Definitions

| Bit(s) | Name | Reset Value | SW Default Value | Description | HAPI Signal |
| --- | --- | --- | --- | --- | --- |
| 0 | PWR_DN | 0 | 0 | Set to power down the IC. IC will wake up on SPI and interrupt activity | NA |
| 1 | SPD_SPSD_CTRL | 0 | 1 | Selects hardware or software control over suspends and speed | TDB (may not be needed since this is not going to be a dynamic setting) |
| 2 | BI_DI | 0 | 0 | Selects Bi-directional or uni-directional transceiver interface | TDB (may not be needed since this is not going to be a dynamic setting) |
| 3-4 | RESERVED |  | 0 | Reserved | NA |
| 5 | AUDIO_EN | 0 | 0 | Enables the audio muxes and disables the USB transceiver when high. | TBD (MUX1 and MUX2 were formerly used for the Audio Mode Selection) |
| 6-7 | RESERVED | 0 | 0 | Reserved | NA |

Table 6-15: MCR2 Bit Definitions

| Bit(s) | Name | Reset Value | SW Default Value | Description | HAPI Signal |
| --- | --- | --- | --- | --- | --- |
| 0 | AUDIO_MONO | 0 | 1 | 0 = Stereo Mode, 1 = Mono Audio | TBD (MUX1 and MUX2 were formerly used for the Audio Mode Selection) |
| 1 | SW_MIC_SPKR_L | 0 | 0 | Audio Loop-back test: 0 = SPKR_L not looped back to MIC, 1 = loop-back enabled | TDB (may not be needed since this is not going to be a dynamic setting) |
| 2 | SW_MIC_SPKR_R | 0 | 0 | Audio Loop-back test: 0 = SPKR_R not looped back to MIC, 1 = loop-back enabled | TDB (may not be needed since this is not going to be a dynamic setting) |
| 3 | ISET_DRV_EN | 0 | 0 | 0 = ISET controlled by hardware, 1 = ISET controlled by ISET_STATE | TBD (could share whatever FACT_DET signal is called as an output) |
| 4 | ISET_STATE | 0 | 1 | Used for SW control over ISET | TBD (mat not be needed, could just be set and use ISET_DRV_EN to control the ISET line) |
| 5 | DP_SRP_EN | 0 | 0 | Enables the D+ pullup resistor regardless of VBUS state. | TBD (HAPI_DP_SRP_EN) |
| 6 | PH_ID_INT | 0 | 0 | Generates a low on ID interrupt for a time Tph_id_wt (covered in the CEA-936 specification). Auto clears | TBD |
| 7 | PH_ID_ACK | 0 | 0 | Generates a low on ID interrupt for a time Tph_id_wt (covered in the CEA-936 specification). Auto clears | TBD |

Table 6-16: ACR Bit Definitions

| Bit(s) | Name | Reset Value | SW Default Value | Description | HAPI Signal |
| --- | --- | --- | --- | --- | --- |
| 0-3 | TMR_DP_INT | 0000b | 0000b | Used to set the time for the DP interrupt for 4 wire CEA-936. Motorola has chosen to use 5 wire so this bit will not be used. | NA |
| 4-8 | TMR_SE1 | 0001b | 0001b | Sets the SE1 detection time in 1ms increments. | TDB (may not be needed since this is not going to be a dynamic setting) |

Table 6-17: TCR Bit Definitions

| Bit(s) | Name | Reset Value | SW Default Value | Description | HAPI Signal |
| --- | --- | --- | --- | --- | --- |
| 0 | DP_PULLUP | 1 | 0 | Enables the 1.5 D+ pull up resistor if VBUS is present | PCAP1_HAPI_USB_VCCRENB |
| 1 | DP_WKPU_EN | 01 | 1 | Enables the charger detection pull up | !HAPI_CHRG_DET_PU_B |
| 2 | DP_PULLDOWN | 0 | 0 | Enables the D+ pull down | NA |
| 3 | DM_PULLDOWN | 1 | 0 | Enables the D- pull down | NA |
| 4 | ID_PULLDOWN | 0 | 0 | Enables the ID pull down | HAPI_SNP_INT_CTL |
| 5 | RESERVED | 0 | 0 | Reserved | NA |
| 6 | VBUS_DISCHRG | 0 | 0 | Discharges VBUS through a pull down | NA |
| 7 | VBUS_CHRG | 0 | 0 | Charge VBUS through a pull up | NA |

1. This is the default state of the initial parts.  Production parts will default to 1.

Table 6-18: RCR Bit Definitions

| Bit(s) | Name | Reset Value | SW Default Value | Description | HAPI Signal |
| --- | --- | --- | --- | --- | --- |
| 0 | VBUS_DET | NA | NA | Indicates VBUS > 3.6V | TBD |
| 1 | SESS_VLD | NA | NA | Indicates VBUS > 2.0V | TBD |
| 2 | DP_HI | NA | NA | Indicates D+ is high | TBD |
| 3 | ID_GND | NA | NA | Indicates the ID line is grounded (SNP_INT) | (HAPI_SNP_INT_B) |
| 4 | SE1 | NA | NA | Indicates a single ended 1 is detected | TBD |
| 5 | ID_FLOAT | NA | NA | Indicates ID pin is floating | (HAPI_PPD_INT_B) |
| 6 | RESERVED | NA | NA | Reserved | NA |
| 7 | DP_INT | NA | NA | Only used in 4 wire implementations | NA |

Table 6-19: ISR Bit Definitions

| Bit(s) | Name | Reset Value | SW Default Value | Description | HAPI Signal |
| --- | --- | --- | --- | --- | --- |
| 0 | VBUS_DET_INT | NA | NA | The bits of the ILR indicate that the state of the corresponding bin in the ISR transitioned through an enabled edge (see IEN_LOW and IEN_HIGH) | TBD |
| 1 | SESS_VLD_INT | NA | NA |  | TBD |
| 2 | DP_HI_INT | NA | NA |  | TBD |
| 3 | ID_GND_INT | NA | NA |  | TBD |
| 4 | SE1_INT | NA | NA |  | TBD |
| 5 | ID_FLOAT_INT | NA | NA |  | TBD |
| 6 | RESERVED | NA | NA |  | TBD |
| 7 | DP_INT_INT | NA | NA |  | TBD |

Table 6-20: ILR Bit Definitions

| Bit(s) | Name | Reset Value | SW Default Value | Description | HAPI Signal |
| --- | --- | --- | --- | --- | --- |
| 0 | VBUS_DET_IEL | 0 | 0 | The bits of the IEN_LOW enable the generation of EMU_INT_B on the falling edge of the corresponding interrupt source. | TBD |
| 1 | SESS_VLD_IEL | 0 | 0 |  | TBD |
| 2 | DP_HI_IEL | 0 | 0 |  | TBD |
| 3 | ID_GND_IEL | 0 | 1 |  | TBD |
| 4 | SE1_IEL | 0 | 0 |  | TBD |
| 5 | ID_FLOAT_IEL | 0 | 1 |  | TBD |
| 6 | RESERVED | 0 | 0 |  | TBD |
| 7 | DP_INT_IEL | 0 | 0 |  | TBD |

Table 6-21: IEN_LOW Bit Definitions

| Bit(s) | Name | Reset Value | SW Default Value | Description | HAPI Signal |
| --- | --- | --- | --- | --- | --- |
| 0 | VBUS_DET_IEH | 0 | 0 | The bits of the IEN_HIGH enable the generation of EMU_INT_B on the rising edge of the corresponding interrupt source. | TBD |
| 1 | SESS_VLD_IEH | 0 | 0 |  | TBD |
| 2 | DP_HI_IEH | 0 | 0 |  | TBD |
| 3 | ID_GND_IEH | 0 | 1 |  | TBD |
| 4 | SE1_IEH | 0 | 0 |  | TBD |
| 5 | ID_FLOAT_IEH | 0 | 1 |  | TBD |
| 6 | RESERVED | 0 | 0 |  | TBD |
| 7 | DP_INT_IEH | 0 | 0 |  | TBD |

Table 6-22: IEN_HIGH Bit Definitions

## Statically Configured GPIO

These signals retain the same GPIO configuration regardless of the operating mode of the bus.  This section serves as a quick reference for the GPIO connectivity; signal definitions and usage are covered in other areas of this specification.  Signals that have the same functionality as Razor are not covered in the ISP1109 addendum section of this ICD.

| GPIO Pin | Signal(s) | Neptune Module | Reuse From Razor |
| --- | --- | --- | --- |
| PA6 | SW_BP_EN | MCU GPIO | Y |
| PA12 | EMU_INT_B | EXT INT 3 | N |
| PA13 | MID_RATE_CTRL | MCU GPIO | Y |
| PD8 | ISP1109_CS | MQSPI (CS2) | N |
| PD10 | USB_TXENB | USB | Y |
| PD11 | USB_VPIN | USB | Y |
| PD13 | USB_XRXD | USB | Y |
| PD15 | USB_SE0 | USB | Y |
| PE12 | ISET_SENSE | MCU GPIO | (Y) |

Table 6-23: Statically Configured GPIO

## Dynamically Configured GPIO

The dynamically configured IOs are identical to Razor.

## Interrupt Handling

Interrupt handling differs slightly from the discrete solution when using the ISP1109.  PCAP2 will still be used for the MOBPORTB interrupt (SPD detection) while the PPD_INT_B and SNP_INT_B signals will be replaced by one interrupt, EMU_INT_B, generated by the ISP1109.  When this interrupt is detected software must poll the ILR for the active source and then use the ISR for any subsequent debouncing.  The interrupt handler for the ISP1109 should be architecturally very similar to the PCAP interrupt handler.

## Bus Configuration

All bus configuration (Audio, USB, RS-232) is implemented through the ISP1109 SPI bits UART_EN, AUDIO_EN, and AUDIO_MONO.  The table below details the settings for the various bus modes.

| Mode | UART_EN | AUDIO_EN | AUDIO_MONO |
| --- | --- | --- | --- |
| USB mode | 0 | 0 | X |
| UART mode | 1 | 0 | X |
| Mono headset / carkit | 0 | 1 | 1 |
| Stereo mode | 0 | 1 | 0 |

Table 6-24: Bus Configuration Settings

## Factory Mode Detection and Factory Mode

Factory mode detection differs from the discrete implementation slightly.  With the ISP1109 solution the ISET_SENSE (formerly FACT_DET) line must be read.  If a high is detected then the SE1 bit of the ISR must be checked.  If low then the radio should enter factory mode. Logically:

Factory mode = (ISET_SENSE && !SE1)

Once factory mode has been detected the DP_SRP_EN bit should be set to ensure USB communication remains uninterrupted.

## External Power Path Control

The eternal power path is controlled by software through the use of the ISET_DRV_EN and ISET_STATE bits of the ACR register.  In order to reduce the number of writes required to the ISP1109 it is suggested the ISET_STATE bit be initialized high and the ISET_DRV_EN be used to control the state of the pin.

## Factory Considerations

### Test Command Requirements

Though there are no hard requirements for new test commands with the ISP1109, new test commands could be spec’ed to ease factory implementation.  All verification can be completed using the SPI read and write commands.  Potential test command additions would be those that abstract the SPI reads/writes to the individual bits of the ISP1109

### Turn On

The radio turn-on sequence should match that of Razor.

### EMU_AUDIO Test Command Changes

Due to the nature of integrating functionality the specifics of this test command will change when using the ISP1109.  These changes should be abstracted in HAPI (MUX lines changing to the audio control lines within the ISP1109).  The modified sequence for the EMU_AUDIO test command is shown below.

Figure 6-19: EMU_AUDIO Test Command (Radio Perspective)

The ISP1109 also includes the ability to loop-back audio internal to the radio which allows testing of the audio connectivity with no external support required.  This is the preferred method for testing audio connectivity going forward; however this method will require a design spanning both the DSP and MCU domains.  Due to the extensive design required for this method it will most likely not be ready during the life of the ISP1109.

### ID Line Test coverage

Since the interrupt generation and control of the ID line is integrated into the ISP1109 the factory testing of the ID line can be reduced to ensuring in spec A/D’s and connectivity to the ISP1109.  This test reduction is based on the assumption the IC is a known good part.

Figure 6-20: ID Line Test Coverage for ISP1109 Based Systems