========

SEEM 0032_0001

bits: 00000000 <-- binary

76543210 <-- bit #

**all offsets are HEX**

offset 00 -	bit 0: Settings > Initial Setup > Time and Date > Autoupdate Menu (1 on, 0 off)

bit 1: Settings > Initial Setup > Time and Date > date setting option (1 on, 0 off)

bit 2: Settings > Initial Setup > Auto Redial (1 on, 0 off)

bit 6: Settings > Phone Status > Other Information (1 on, 0 off)

bit 7: Settings > Phone Status > Other Information > S/W Version, DSP S/W Version, Language List (1 on, 0 off)

offset 01 -	bit 0: Settings > Phone Status > Other Information > PRL Version (1 on, 0 off)

bit 1: Settings > Phone Status > Other Information > Data Ready (1 on, 0 off)

bit 2: Settings > Phone Status > Other Information > TrueSync Ready (1 on, 0 off)

bit 3: Settings > Phone Status > Other Information > Browser Ready (1 on, 0 off)

bit 5: SIM capable device (1 on, 0 off)

offset 02 -	bit 0: Settings > In-Call Setup > Answer Options > Open To Answer (1 on, 0 off)

bit 1: BLOCKS THESE FEATURES: Call barring, Call transfer, Info Services, do not read VM number from SIM, Voicemail ^@ text messages, Multi-Line Support, My Network List, Manual Network Search (1 on, 0 off - set to 0 to UNBLOCK these features)

some of these features may be SIM dependant anyway, so activating it on your phone may not actually allow you to use it

bit 4: ^@ and _@ VM Indicator SMS Msgs (1 off, 0 on) (**DOES NOT WORK RELIABLY**)

bit 7: Message > Info Services (1 on, 0 off)

offset 03 -	bit 1: Message > Quick Notes (1 on, 0 off)

bit 2: Message > Drafts (1 on, 0 off)

bit 3: Message > Outbox (1 on, 0 off)

bit 5: Message > Inbox (1 on, 0 off)

bit 6: Settings > Ring Styles > Details > Ans Mach option (1 on, 0 off)

bit 7: Alarms Available (1 on, 0 off)

offset 04 - 	bit 1: Call Restriction Support (1 on, 0 off)

offset 05 - 	bit 3: Silent alert (ringtones) (1 on, 0 off)

offset 06 - 	bit 1: Siren alert (1 on, 0 off)

offset 08 -	bit 2: Office Tools > Voice Records (1 on, 0 off)

bit 3: Allow Voicenote for Datebook (1 on, 0 off)

bit 4: Allow Voicenote for Ring Alerts (1 on, 0 off)

bit 5: Recent Calls > Call Cost (1 on, 0 off)

bit 7: Phonebook > Edit > Voice Name (1 on, 0 off)

offset 09 - 	bit 0: Voice Dialing Support (1 on, 0 off)

bit 1: Settings > Security > Talk Secure (1 on, 0 off)

bit 3: Settings > Security > Lock Application > Recent Calls (1 on, 0 off)

bit 4: Settings > Security > Lock Application > Phonebook (1 on, 0 off)

bit 5: Settings > Security > Lock Application > Datebook (1 on, 0 off)

bit 6: Settings > Security > Lock Application > Radio (1 on, 0 off)

bit 7: Settings > Security > Lock Application > Messaging (1 on, 0 off)

offset 0A - 	bit 0: Settings > Security > Lock Application > Shortcuts (1 on, 0 off)

bit 1: Settings > Security > Lock Application > SIM Apps (1 on, 0 off)

bit 2: Settings > Security > Lock Application > Browser (1 on, 0 off)

bit 3: Phonebook > Edit > Insert P (Pause) (1 on, 0 off)

bit 4: Phonebook > Edit > Insert W (Wait) (1 on, 0 off)

bit 5: Phonebook > Edit > Insert N (Number) (1 on, 0 off)

bit 6: iTAP Support (1 on, 0 off)

bit 7: Settings > Initial Setup > Backlight > Continuous option (1 on, 0 off)

offset 0B -	bit 0: Settings > Phone Status > Other Information > ESN (1 on, 0 off)

bit 2: GSM Phone Available (1 on, 0 off)

bit 3: Automatic Lanaguage Support (1 on, 0 off)

offset 0C -	bit 0: Multiple Language Support (1 on, 0 off)

bit 1: Menu: Smart Key (1 on, 0 off)

bit 6: Menu: Services (1 on, 0 off)

offset 0D -	bit 4: Recent Calls > Detail (1 on, 0 off)

bit 7: Settings > Connection > Bluetooth (1 on, 0 off) (menu item, may require update to mma files)

offset 0E -	bit 0: Retrieve IMEI by pressing *#06# (1 on, 0 off)

bit 1 - Subsidy Lock Available (1 on, 0 off)

bit 4 - GSM 900MHz Capable (1 on, 0 off)

bit 5 - GSM 1800MHz Capable (1 on, 0 off)

bit 6 - GSM 1900MHz Capable (1 on, 0 off)

offset 0F - 	bit 0: Browser Additional Charge Prompt (1 on, 0 off)

offset 13 - 	bit 0: Multimedia > Radio (1 on, 0 off)

bit 6: "City" Displayed when in certain areas (provider specific, O2 Germany) (1 on, 0 off)

offset 14 - 	bit 0: Settings > In-Call Setup > My Caller ID > All Calls (1 on, 0 off)

bit 1: Office Tools > SIM Applications (1 on, 0 off)

bit 2, 3: Office Tools > Dialing Services > Quick Dial (1 on, 0 off) (bit 2 can cause Settings > Personalize menu to hang) (menu item, may require update to mma files)

bit 4: Settings > Network > Current Network (1 on, 0 off)

bit 5: Settings > Network > New Network (1 on, 0 off)

bit 6: Settings > Network > Network Setup (1 on, 0 off)

bit 7: Settings > Network > Search Type (1 on, 0 off)

offset 15 - 	bit 2: Settings > Network > Available Networks (1 on, 0 off)

bit 3: Settings > Network > My Network List (1 on, 0 off)

bit 4: Settings > Network > Service Tone (1 on, 0 off)

bit 5: Settings > Network > Call Drop Tone (1 on, 0 off)

bit 7: Allow CALL BAR Password Change (1 on, 0 off)

offset 16 - 	bit 6: Settings > Ring Styles > Details > Roam Calls option, special ring tone for calls received while roaming (1 on, 0 off)

bit 7: Allow Numeric Text Entry (1 on, 0 off)

Offset 17 -	bit 2: Allow TAP Text Entry (1 on, 0 off)

bit 6: Settings > Phone Status > Other Information > Technology (1 on, 0 off)

bit 7: GPRS Capable (1 on, 0 off)

offset 18 - 	bit 0: Browser Display CSD Dialogue (Warn User of CSD Call) (1 on, 0 off)

bit 4: WebAccess > Browser > Menu > Set Homepage (1 on, 0 off)

bit 5: WebAccess > Browser Setup (1 on, 0 off)

bit 6: Webaccess > Web Sessions > Edit > UserID (1 on, 0 off)

bit 7: Webaccess > Web Sessions > Edit > Password (1 on, 0 off)

offset 19 -	bit 0: Webaccess > Web Sessions > Edit > CSD (1 on, 0 off)

bit 1: Webaccess > Web Sessions > Edit > Timeout (1 on, 0 off)

bit 2: Webaccess > Web Sessions > Edit > Line Type (1 on, 0 off)

bit 3: WebAccess > Browser Setup > Confirmation option (1 on, 0 off)

bit 4: WebAccess > Browser Setup > Cache Startup (1 on, 0 off)

offset 1A -	Bit 0: CSD Capable (Circuit Switched Dialing) (1 on, 0 off)

Bit 2: Browser: CSD Errors are Transparent (1 on, 0 off)

Bit 6: Browser: Homepage Available (1 on, 0 off)

offset 1B -	bit 0: Browser: OTA Provisioning Available (1 on, 0 off)

bit 3: Media Center > Incoming Call (1 on, 0 off)

bit 5: SIMTK Sustain Call (1 on, 0 off)

bit 6: Settings > Initial Setup > Auto PIN Dial (1 on, 0 off)

bit 7: Settings > Initial Setup > 1-Touch Dial (1 on, 0 off)

offset 1C -	bit 0: Settings > Initial Setup > Preferred Card (1 on, 0 off)

bit 3: Browser: Standard Provisioning (1 on, 0 off)

bit 6: Settings > Connection > IrDA available (1 on, 0 off)

bit 7: Settings > In-Call Setup > My Caller ID (1 on, 0 off)

offset 1D -	bit 0: External LCD Backlight (1 always on when flip closed, 0 on briefly when flip closed)

bit 3: Moto Startup Animation (1 on, 0 off)

bit 4: Security/PIN Code Support (1 on, 0 off)

bit 5: In-Call: Speed Dial (1 on, 0 off)

bit 6: Settings > Initial Setup > Battery Save (1 on, 0 off)

offset 1E - 	bit 0: Keypad: Voicerecord Key Available (Side Key) (1 on, 0 off)

bit 2: Bluetooth Headset Support (1 on, 0 off)

bit 3: Settings > Initial Setup > Animation (1 on, 0 off)

bit 6: Message > Menu > Message Setup > Expire After (1 on, 0 off)

bit 7: Settings > In-Call Setup > Talk and Fax > All Calls (1 on, 0 off)

offset 1F -	bit 0: Settings > In-Call Setup > Call Waiting (1 on, 0 off)

bit 1: CSD (Circuit Switched Dialing) (1 on, 0 off)

bit 2: General Cell Broadcast (1 on, 0 off)

bit 3: Message > Info Services Setup (1 on, 0 off)

bit 4: Settings > Phone Status > Active Line (Multi-Line Support) (1 on, 0 off)

bit 5: Settings > Security > Call Barring (1 on, 0 off)

bit 6: Data Calls (1 on, 0 off)

bit 7: Fax Calls (1 on, 0 off)

offset 20 -	bit 0: Messages > Voicemail (1 on, 0 off)

bit 1: Settings > In-Call Setup > Talk and Fax (1 on, 0 off)

bit 6: Recent Calls > Call Cost configuration (1 on, 0 off)

offset 22 -	bit 5: Settings > Call Divert/Forward > Voice Calls (1 on, 0 off)

bit 6: Settings > Call Divert/Forward > Fax (1 on, 0 off)

bit 7: Settings > Call Divert/Forward > Data Calls (1 on, 0 off)

offset 23 - 	bit 1: In-Call: Call Hold (1 on, 0 off)

bit 2: Settings > Call Forward (1 on, 0 off)

bit 5: Settings > In Call Setup > Call Guard option (1 on, 0 off)

offset 24 -	bit 2: Press & Hold 1 to Autodial Voice Mail Number (1 on, 0 off)

bit 3: Custom Startup Sound (004a_0001, offset 22 must be set to 01) (1 on, 0 off)

offset 26 - 	bit 2: v.42BIS Compression Available for CSD (1 on, 0 off)

bit 5: GSM Dual-Band Phone (1 on, 0 off)

offset 27 - 	bit 4 - MT CSD Support (1 on, 0 off)

offset 28 - 	bit 5: Phonebook: Edit Name (1 on, 0 off)

bit 6: Phonebook: Edit Number (1 on, 0 off)

offset 29 -	bit 1: Settings > Security > Lock Keypad (1 on, 0 off)

bit 2: Menu + # > My Phonenumber (1 on, 0 off)

offset 2B - 	bit 0: Test Mode On (1 on, 0 off) (allows "menu 0 0 * * T E S T M O D E *" to enable field test mode)

bit 1: Test Mode Off (1 on, 0 off) (allows "menu 0 0 * * T E S T O F F *" to disable field test mode)

bit 3: Settings > Initial Setup > Language (1 on, 0 off)

bit 4: Settings > Initial Setup > Contrast (1 on, 0 off)

bit 7: Allow Lock-Code Change (1 on, 0 off)

offset 2C -	bit 4: Roaming Menu (1 on, 0 off)

offset 2F -	bit 1: WebAccess > Web Sessions Menu (1 on, 0 off) (menu item, may require update to mma files)

bit 2: Webaccess > Web Sessions > Edit > IP1 (1 on, 0 off)

bit 3: Webaccess > Web Sessions > Edit > IP2 (1 on, 0 off)

bit 4: Webaccess > Web Sessions > Edit > CSD1 (1 on, 0 off)

bit 5: Webaccess > Web Sessions > Edit > CSD2 (1 on, 0 off)

bit 6: AUTO APN - Queries GGSN for Default APN if Blank in Websession (1 on, 0 off)

bit 7: Time to hold power button down to power on phone (1 instant, 0 press and hold)

offset 30 -	bit 1: Motorola Greeting Tone Available (1 on, 0 off)

bit 5: Office Tools > Dialing Services > Quick Dial (allow edit) (1 on, 0 off)

bit 6: Settings > Personalize > Home Screen > Banner (1 on, 0 off) (text in 0063_0001)

bit 7: Settings > Personalize > Greeting (1 on, 0 off) (text in 013a_0001)

offset 31 - 	bit 0: Moto Shutdown Animation (1 on, 0 off)

bit 4: Settings > Security > Lock Application > Voice Notes (1 on, 0 off)

bit 5: Settings > Security > Lock Application > Settings (1 on, 0 off)

bit 7: Settings > Security > Lock Application > Roaming (1 on, 0 off)

offset 32 -	bit 0: Settings > Security > Lock Application > Service Dial (1 on, 0 off)

bit 1: Settings > Security > Lock Application > Quick Dial (1 on, 0 off)

bit 2: Settings > Security > Lock Application > Messaging (1 on, 0 off)

offset 33 -	bit 6: Settings > Initial Setup > Backlight > Color (1 on, 0 off)

offset 34 -	bit 3: Display Roam Triangle Icon (1 on, 0 off)

bit 5: Office Tools > Datebook (1 on, 0 off)

bit 6: Settings > Phone Status > Other Information > PRI Checksum (1 on, 0 off)

offset 36 -	bit 2: Settings > Phone Status > My Tel. Numbers (1 on, 0 off)

bit 4: Opcode Menu (Menu->048263*) Available (1 on, 0 off)

offset 37 -	bit 1: *#9999# from home screen shows "Other Information" menu (1 on, 0 off)

bit 2: Settings > Initial Setup > Status Light (1 on, 0 off)

bit 3: Office Tools > Calculator (1 on, 0 off)

offset 38 - 	bit 1: Continental alert (1 on, 0 off)

bit 2: Classic alert (1 on, 0 off)

bit 3: Attention alert (1 on, 0 off)

offset 3B -	bit 2: Vibe Dot alert (1 on, 0 off)

bit 3: Vibe Dash alert (1 on, 0 off)

bit 4: Vibe Dot Dot alert (1 on, 0 off)

bit 5: Vibe Dot Dash alert (1 on, 0 off)

bit 6: Vibe Pulse alert (1 on, 0 off)

offset 3C -	bit 7: SMS Allow Multiple Destinations (1 on, 0 off)

offset 3D -	bit 0: E-Mail via SMS (1 on, 0 off)

bit 1: SMS delivery reports (1 on, 0 off)

bit 3: Message Character Counts (0 for up, 1 for down) (only works on phones with TRIPLETS firmware)

bit 5: EMS via SMS (Enhanced Messaging Service) (1 on, 0 off)

bit 7: Message > Inbox Setup > Email Gateway (1 on, 0 off) see 56/4 also

offset 3E -	bit 4: for purposes of linking name in phonebook with caller ID number, xxx-xxxx = xxx-xxx-xxxx = 1-xxx-xxx-xxxx = +1-xxx-xxx-xxxx (0 on, 1 off)

bit 5: Office Tools > Quick Menu (1 on, 0 off) (menu item, may require update to mma files)

bit 7: Recent Calls > Call Times > Reset (1 on, 0 off)

offset 3F -	bit 4: SMS Unicode (1 on, 0 off)

bit 6: Settings > Security > Restrict Calls (1 on, 0 off)

offset 40 -	bit 1: Display 'GPRS Attached' icon (1 on, 0 off)

bit 3: Display: Show Credit Remaining on Idle (1 on, 0 off)

bit 4: toggles forced display of provider information from SIM (ie "www.fido.ca") in second line (1 on, 0 off)

bit 6: SMS storage (0 for phone, 1 for SIM)

offset 41 -	bit 0: Datebook (C550) (1 on, 0 off)

bit 2: Display Foreign Carrier Name from SIM under Provider Name (1 show date always, 0 show foreign carrier name)

bit 5: Display: GPRS Icon Indicates Current Status (1 on, 0 off)

bit 6: Swap Delete Confirmation Key Position (1 on, 0 off)

bit 7: Allow CLI Send (Calling Line Identifier) (1 on, 0 off)

offset 43 -	bit 0: Settings > Java Settings > Java App Loader (1 on, 0 off)

bit 1: Settings > Java Settings > Delete All Apps (1 on, 0 off)

bit 2: Browser: Browser Msgs (1 on, 0 off)

bit 4: GSM Multi-Band Region Support for Automatic Scan (See Ofs 43 Bit 5) (1 on, 0 off)

bit 5: Settings > Network > Network Setup > Band > Automatic (1 on, 0 off)

bit 6: Settings > Personalize > Screensaver (1 on, 0 off)

bit 7: Multimedia > Pictures (Picture Viewer) (1 on, 0 off)

offset 44 -	bit 0: Settings > Personalize > Wallpaper (1 on, 0 off)

bit 1: Settings > Security > Lock Application > Pictures (1 on, 0 off)

bit 2: SMS: MO Concatenation (1 on, 0 off)

bits 4, 5, 6, 7: Office Tools > Engineering (1 on, 0 off) (menu item, may require update to mma files)

offset 45 -	bit 0: Keypad: Combined Power and End Key (1 on, 0 off)

bit 7: Settings > Phone Status > Other Information > MP3 Ready (1 on, 0 off)

offset 46 -	bit 0: Custom Shutdown Animation in seems 0207/0209 (1 on, 0 off)

bit 3: Messages > Menu > Message Setup > Message/Service Center Number (1 on, 0 off)

bit 4: Messages > Menu > Voicemail Setup (1 on, 0 off)

bit 6: Office Tools > Chat (1 on, 0 off) (menu item, may require update to mma files)

bit 7: Datebook Alarms (1 on, 0 off)

offset 47 -	bit 0: Settings > Security > Lock Application > Calculator (1 on, 0 off)

bit 1: Settings > Security > Lock Application > Games and Apps (1 on, 0 off)

bit 3: Settings > Initial Setup > TTY (1 on, 0 off)

bit 7: SMS CAPS lock (1 ABC, 0 Abc)

offset 48 -	bit 1: Display My Phone Number at Power On (1 on, 0 off) (see SEEM 0126_001 for S/W .38R and above)

bit 3: Browser > Menu > Bookmarks (1 on, 0 off)

bit 4: Settings > Call Divert/Forward > Voice Calls > Conditional (1 on, 0 off)

bit 5: Settings > Call Divert/Forward > Voice Calls > Busy (1 on, 0 off)

bit 6: Settings > Call Divert/Forward > Voice Calls > No Answer (1 on, 0 off)

bit 7: Settings > Call Divert/Forward > Voice Calls > Not Reachable (1 on, 0 off)

offset 49 -	bit 2: Right Soft Key Edit Available (1 on, 0 off)

bit 3: SMS forwarding header (prepends to subject line when forwarding a message) (1 add header, 0 no header)

bit 5: Web Access > Browser custom animated splash screen stored in /a/mib_op_greet (1 on, 0 off) (if set to 0, offset 1D, bit 3 will control whether or not a default motorola animation will display)

bit 7: Keypad: 4-Way Navigation Key (1 on, 0 off)

offset 4A -	bit 0: Settings > Personalize > Banner Graphic (1 on, 0 off)

bit 1: Message > E-Mail Msgs (1 on, 0 off)

bit 4: Settings > Security > Lock Application > Ring Styles (1 on, 0 off)

bit 5: Display: SMS Show Sender ID First on Preview (1 on, 0 off)

bit 6: toggles display of "Emergency Only" when a home or ok-to-roam network cannot be found, but another network is available (1 on, 0 off)

bit 7: toggles display of "No Service" when network cannot be found (1 on, 0 off) (this fixes the "No Service" display overwriting the time on the exterior LCD)

offset 4B -	bit 0: Recent Calls > Data Times (1 on, 0 off) (offset 4B, bit 3 must be set to 1)

bit 1: Recent Calls > Data Volumes (1 on, 0 off) (offset 4B, bit 3 must be set to 1)

bit 2: Byte Usage Summary at End of GPRS Session (xxx bytes transferred) (1 on, 0 off)

bit 3: Activate Recent Calls > Data counters (1 activated, 0 deactivated) (must activate specific menu items at offset 4B, bits 0 and 1)

bit 4: Recent Calls > Data Times/Volumes > Recent/All Data Sessions > Internal Data Session (1 on, 0 off) (offset 4B, bit 3 must be set to 1; offset 4B, bit 0, 1, or both must also be set to 1)

bit 5: Settings > Security > Lock Application > Chat (1 on, 0 off)

bit 6: Alpha Tag Display (1 from SIM, 0 from phone)

bit 7: Settings > Security > Lock Application > Call Divert/Forward (1 on, 0 off)

offset 4C -	bit 0: Settings > Security > Lock Application > Phone Status (1 on, 0 off)

bit 1: Settings > Security > Lock Application > Connections (1 on, 0 off)

bit 2: Settings > Security > Lock Application > In-Call Setup (1 on, 0 off)

bit 3: Settings > Security > Lock Application > Personalise (1 on, 0 off)

bit 4: Settings > Security > Lock Application > Initial Setup (1 on, 0 off)

bit 5: Settings > Security > Lock Application > Car Settings (1 on, 0 off)

bit 6: Settings > Security > Lock Application > Headset (1 on, 0 off)

bit 7: Settings > Security > Lock Application > Audio Player (1 on, 0 off)

offset 4D -	bit 0: Settings > Security > Lock Application > Video Player (1 on, 0 off)

bit 1: Settings > Security > Lock Application > Security (1 on, 0 off)

bit 2: Settings > Security > Lock Application > Network (1 on, 0 off)

bit 4: Browser: Numbered Options Support (Press 3 for Item 3 etc.) (1 on, 0 off)

bit 5: Browser: Send SI SL In Accept Headers (Required for WAP 2.0) (1 on, 0 off)

bit 6: Browser: Accept Headers Control Available (Set to OFF for T-Mobile US) (1 on, 0 off)

offset 4F -	bit 2: CHAP over GPRS Support (1 on, 0 off)

bit 6: Browser: Allow Back Button on Blank WML Page (1 on, 0 off)

bit 7: Browser: Disallow Browser Content Forwarding (1 on, 0 off)

offset 50 -	bit 0: 850 MHz Capable (1 on, 0 off)

bit 1: Webaccess (1 on, 0 off)

bit 7: Settings > In Call Setup > MSG Alert option (1 on, 0 off)

offset 51 -	bit 2: Messaging > Menu > Reorder (1 on, 0 off)

bit 3: Messaging > Create Message (1 on, 0 off)

bit 5: Multimedia > Motomixer (1 on, 0 off) (menu item, may require update to mma files)

bit 6: Voice Commands (use Menu+voice key) (0 on, 1 off)

bit 7: Multimedia > Themes (1 on, 0 off) (menu item, may require update to mma files)

offset 52 - 	bit 0: Multimedia > Themes > Setup > Delete (1 on, 0 off)

bit 6: Office Tools > AIM (1 on, 0 off) (menu item, may require update to mma files)

bit 7: Settings > Security > Lock Application > AIM (1 on, 0 off)

offset 53 -	bit 1: MT EMS Message Allow MIDI (1 on, 0 off)

bit 3: Show Personalize Phone Dialog on Reset or New SIM (1 on, 0 off)

bit 4: Settings > Security > Lock Application > Webaccess (1 on, 0 off)

bit 6: Settings > Personalize > Color Styles (allows for custom color style, upload spcolor.sch to /a/mobile/system/) (1 on, 0 off)

bit 7: Settings > Security > Lock Application > Skins/Color Style (1 on, 0 off)

offset 54 -	bit 0: Settings > Personalise > Skins/Color Style (1 on, 0 off)

bit 1: Default Skin/Color Style Available (1 on, 0 off)

bit 2: Office Tools > Mobile QQ (1 on, 0 off) (menu item, may require update to mma files)

bit 3: Settings > Security > Lock Application > Mobile QQ (1 on, 0 off)

bit 4: Webaccess > Bookmarks: Shorcuts: 50 URL's Supported (1 on, 0 off)

bit 5: Delete Confirmation (1 on, 0 off)

bit 6: function of right soft key when reading SMS message (0 delete, 1 reply)

bit 7: BLOCK ADVANCED MENUS: Settings > Network (1 on, 0 off - set to 0 to UNBLOCK advanced Network Menus)

offset 55 - 	bit 2: Time Autoupdate (1 on, 0 off) <-- this will actually activate the feature, the menu item is in offset 00

bit 3: HTTP OTA Client (1 on, 0 off)

bit 4: Settings > Initial Setup > Time and Date (Allow Change) (1 on, 0 off)

bit 5: GSM Multi-Band Phone (1 on, 0 off)

offset 56 - 	bit 1: Browser: Disable Store as Wallpaper for Graphics (1 on, 0 off)

bit 2: Browser: Browser > Menu > Home (1 on, 0 off)

bit 4: Messages > Inbox Setup > E-Mail Gateway (1 on, 0 off) see 3D/7 also

bit 6: SIM Language Support (1 on, 0 off)

offset 57 -	bit 2: Reduced Ringback (Redial) (1 on, 0 off)

offset 58 -	bit 1: Office Tools (1 on, 0 off)

bit 4: Office Tools > Shortcuts, list web shortcuts also (1 don't list, 0 list them)

bit 5: URL Shortcut Edit (1 on, 0 off)

bit 6: Phonebook > Menu > Copy > All to SIM/Phone Memory (1 on, 0 off)

offset 59 -	bit 4: Office Tools > Alarm Clock (1 on, 0 off)

bit 5: Settings > Security > Lock Application > Alarm Clock (1 on, 0 off)

bit 7: ITAP: Smart Punctuation (1 on, 0 off)

offset 5A -	bit 1: Display: -More- Available (1 on, 0 off)

bit 3: Display: Picture Caller-ID (1 on, 0 off)

bit 6: WebAccess > Stored Pages Menu (1 on, 0 off) (menu item, may require update to mma files)

bit 7: Browser: Browser Callback (1 on, 0 off)

offset 5B -	bit 0: WebAccess > Web Shortcuts (1 on, 0 off) (menu item, may require update to mma files)

bit 2: WebAccess > Browser History (1 on, 0 off) (menu item, may require update to mma files)

bit 3: Browser: Browser > Menu > Reload (Reload URL) (1 on, 0 off)

bit 6: Browser: Copy Text (1 on, 0 off)

bit 7: Browser: Copy All (1 on, 0 off)

offset 5C - 	bit 0: Browser: Browser > Menu > Exit (1 on, 0 off)

bit 1: Browser: Browser > Menu > View URL (1 on, 0 off)

bit 2: WebAccess > Go to URL Menu (1 on, 0 off) (menu item, may require update to mma files)

bit 4: Browser: Use Cookies (1 on, 0 off)

bit 6: Phone Wakeup for Alerts (1 on, 0 off)

bit 7: Message > Browser Msgs > Menu > Brwsr Msg Setup > Msg Notification > Medium Priority (1 on, 0 off) (Required for T-Mobile USA customers utilizing the E-Mail Alerts Function)

offset 5D -	bit 1: Settings > Personalize > Ring Styles > Silent Details (1 on, 0 off)

offset 5E -	bit 5: MMS: MMS Available (1 on, 0 off)

bit 6: MMS > New Multimedia Message > Receipt > Delivery Report (1 on, 0 off)

bit 7: MMS > New Multimedia Message > Receipt > Read Report (1 on, 0 off)

offset 5F -	bit 1: Message > Menu > Message Setup > MMS Message Setup > Server Info, Activate Menu Key for New, Edit, Delete (1 on, 0 off)

bit 2: Message > Inbox Setup > MMS > Server Info > Edit (1 on, 0 off)

bit 3: Display: Show Sender ID on Summary View (1 on, 0 off)

bit 4: Personalize Right Soft Key (1 on, 0 off)

bit 5: Personalize Left Soft Key (1 on, 0 off)

bit 6: Webaccess > Web Sessions > Menu > Set Default (1 on, 0 off)

bit 7: Multimedia (1 on, 0 off)

offset 60 -	bit 0: 'GPRS Active' text with GPRS PDP Context Icons (1 on, 0 off)

bit 1: Settings > Java Settings > DNS IP viewable (1 on, 0 off)

bit 2: Settings > Java Settings > DNS IP changable (1 on, 0 off)

bit 6: Video Camera Support (1 on, 0 off)

bit 7: Settings > Security > Lock Application > Multimedia (1 on, 0 off)

offset 61 -	bit 1: External Storage Devices Available: MMC or SD Card Slot (1 on, 0 off)

bit 3: Office Tools > Datebook > Weekview (1 on, 0 off)

bit 4: Office Tools > Datebook > Monthview (1 on, 0 off)

offset 62 -	bit 1: WAP Support (1 on, 0 off)

bit 3: Settings > Connection > Sync (1 on, 0 off) (menu item, may require update to mma files)

bit 4: SMS: SMS Over GPRS Protocol Enable (1 on, 0 off)

bit 5: Messages > Message Setup > Use GPRS (1 on, 0 off)

bit 7: Browser: Dial Prompt Available (1 on, 0 off)

offset 63 -	bit 0: WebAccess > Web Sessions (1 on, 0 off)

bit 2: Settings > Ring Styles > Ring Lights (1 on, 0 off)

bit 4: Settings > Security > Lock Application > IM (1 on, 0 off)

offset 64-	bit 2: SIMTK In-Call Menu (1 on, 0 off)

bit 3: E-Mail WAP Push (1 on, 0 off)

bit 4: E-Mail SSL Support (1 on, 0 off)

bit 5: RE: In Subject (1 on, 0 off)

bit 6: FWD: In Subject (1 on, 0 off)

offset 65 -	bit 0: Settings > Ring Styles > Event Lights (funlight.pat required in /a/mobile/audio/) (1 on, 0 off)

bit 1: Office Tools > IM (1 on, 0 off) (menu item, may require update to mma files)

bit 2: Replace HOLD with MUTE Softkey (1 on, 0 off)

bit 3: MMS Invisible Downloads (Background) (1 on, 0 off)

bit 4: WebAccess/Internet (1 on, 0 off)

bit 6: Phonebook: Picture-ID Listings (1 on, 0 off)

bit 7: DRM FWD Lock and Combined Delivery (1 on, 0 off)

offset 66 -	bit 0: DRM Seperate Delilvery (1 on, 0 off)

bit 1: HTTP Capable (1 on, 0 off)

bit 2: SIMTK Set Playtone Volume (1 on, 0 off)

bit 3: Resize Images (1 on, 0 off)

bit 5: MMS Kb Counter (1 on, 0 off)

bit 6: SMIL Message Composer (1 on, 0 off)

bit 7: Soundtracks (1 on, 0 off)

offset 67 -	bit 4: Display call divert/forward icon on line 1 and status on line 3 (1 on, 0 off) (must be on if offset 02, bit 4 is on)

bit 6: Display Thumbnails in Multimedia > Pictures (1 on, 0 off)

bit 7: Send To New Phonebook Entry (1 on, 0 off)

offset 68 -	bit 0: Settings > Initial Setup > Vibe Sync (1 on, 0 off)

bit 2: MMS Postcard (1 on, 0 off)

bit 3: MMS Templates Available (1 on, 0 off) (see also offset 6B, bit 7)

bit 4: GPRS Packet Data Transfer Icon (1 on, 0 off)

bit 6: Message > Create Message > New Letter (1 on, 0 off)

offset 69 -	bit 3: Multi-page SMS Send Confirmation (greater than 160 characters) (1 on, 0 off)

bit 4: File Manager (1 on, 0 off)

bit 5: Insert Phonebook Number In Msg Body (1 on, 0 off

offset 6A -	bit 0: Audio Player (1 on, 0 off)

bit 1: Multimedia > Pictures > Picture Viewer > Resolution (1 on, 0 off)

bit 3: Settings > Initial Setup > Display Timeout (1 on, 0 off)

bit 4: Multimedia > Pictures > Picture Viewer > Slideshow (1 on, 0 off)

bit 5: Multimedia > Sounds > Audio Player > Playlist (1 on, 0 off)

bit 6: Multimedia > Auto Timer (Sounds and Pictures) (1 on, 0 off)

bit 7: PC to Phone Transfer (1 on, 0 off)

offset 6B -	bit 0: BlueTooth OBEX (1 on, 0 off)

bit 1: Browser: Delete Cache On Exit (1 on, 0 off)

bit 4: default curser position on main menu when viewed by icon (1 center, 0 top left)

bit 5: GPRS Packet Data Icon (1 on, 0 off)

bit 6: VM message counter in Messages > Voicemail (0 on, 1 off)

bit 7: Message > MMS Templates (1 on, 0 off) (see also offset 68, bit 3)

offset 6C -	bit 0: Keypad: 5-Way Nav Key (1 on, 0 off)

bit 1: WebAccess > Browser (show address bar) (1 on, 0 off)

bit 3: BLOCK/LIMIT: Settings > Personalize > Home Screen > Home Keys (limited), Layout (blocked) (1 on, 0 off - set to 0 to UNBLOCK/UNLIMIT these features)

bit 5: Multimedia > Camera > Brightness (1 on, 0 off)

offset 6D -	bit 5: Silent Alerts Set At 0 (1 on, 0 off)

bit 7: Cingular Custom Prompts (1 on, 0 off)

offset 6E - 	bit 1: remove right soft key on home screen for custom branding (ie. Vodaphone Live!) (1 on, 0 off)

bit 2: Multimedia > Video > Menu > Videos Setup > Recording Sound (1 on, 0 off)

bit 3: Multimedia > Video > Menu > Videos Setup > Length (1 on, 0 off)

bit 4: Multimedia > Video (1 on, 0 off) (menu item, may require update to mma files)

offset 6F -	bit 0: Multimedia > Camera > Zoom (1 on, 0 off)

bit 2: Multimedia > Videos/Pictures/Sounds > Auto Repeat (1 on, 0 off)

bit 3: Multimedia > Videos/Pictures/Sounds > Shuffle (1 on, 0 off)

bit 4: Multimedia > Sounds/Videos > Menu > Sounds/Videos Setup > Timer (1 on, 0 off)

bit 5: Multimedia > Pictures > Menu > Pictures Setup > Counter (1 on, 0 off)

bit 6: Multimedia Allow Alerts (1 on, 0 off)

bit 7: Multimedia > Video > Menu > Videos Setup > Video Quality (1 on, 0 off)

offset 70 -	bit 0: Multimedia > Video > Menu > Videos Setup > Lighting Condition (1 on, 0 off)

bit 1: Multimedia > Camera/Video > Setup > Exposure (1 on, 0 off)

bit 2: Multimedia > Camera > Setup > Shutter Tone (1 on, 0 off)

bit 3: Multimedia > Camera > Setup > Mirror (1 on, 0 off) (can cause 'picture failed' errors)

bit 4: Multimedia > Camera > Menu > Pictures Setup > Status Icons (1 on, 0 off)

bit 5: Allow Shutter Tone NONE Option (1 on, 0 off)

bit 6: Multimedia > Video Camera (1 on, 0 off)  (see also seem 004a, offset 1D3) (menu item, may require update to mma files)

offset 71 -	bit 0: Vibrate Fix for V180 Phones Flashed with V220 Firmware

bit 1: Browser (Only if No WebAccess Supported Menu) (1 on, 0 off)

bit 4: Settings > Java Settings > Switching to Debug option (1 on, 0 off)

bit 6: Games & Apps display splash screen (1 on, 0 off)

bit 7: Settings > Java Settings > Start TCK (1 on, 0 off)

offset 72 - 	bit 0: J2ME: Tell A Friend Option (Games and Apps) (1 on, 0 off)

bit 1: Settings > Java Settings > App Priority, Set Standby App, App Backlight/Blink (1 on, 0 off)

bit 5: Browser: Cache (1 on, 0 off)

bit 6: Multimedia: Image Style Support (1 on, 0 off)

bit 7: Dynamic Prompts (/a/dnp.prf) changes message names (0 for 'Short Msg, MMS' , 1 for 'Text Msg, Multimedia') **FLEX Dependent

offset 73 -	bit 1: Camera > Store Options > Send to My Album (1 on, 0 off) (**OPERATOR / FLEX Dependent**)

bit 7: Settings > Security > Lock Application > BlueTooth (1 on, 0 off)

offset 74 - 	bit 4: provider tag (1 to remove, 0 to display) (offset 4B, bit 6 must be set to 0) (removes justification on home screen, Settings > Personalize > Home Screen > Layout, causes lines of 000-000 to display at Settings > Network > Available Networks)

bit 6: Settings > Initial Setup > Brightness (1 on, 0 off)

offset 75 -	bit 4: Settings > Call Forwarding 'detailed' option added (1 on, 0 off)

bit 5: Settings > Initial Setup > Display Timeout > 5 and 10 minutes (0 on, 1 off)

bit 7: Display MMS Progressbar (1 on, 0 off)

offset 76 -	bit 2: Allow MSG Edit On Resend (1 on, 0 off)

bit 3: Hide Edit On Resend Confirmation Dialogue (1 on, 0 off)

bit 4: Remove TO: Field From Reply (1 on, 0 off)

bit 5: Show Web Browser Loading Status Animations (1 on, 0 off)

bit 7: Browser: Title Scroll Support (1 on, 0 off)

offset 77 -	bit 0: Browser: URL as Title (1 on, 0 off)

bit 6: Multimedia > Camera (1 on, 0 off)

offset 78 -	bit 0: Display MMS Status Pop-Up (1 on, 0 off)

bit 3: Multimedia > Pictures > Menu > Send as TIM Click (1 on, 0 off) (language pack dependent)

offset 79 -	bit 1: Keypad: Camera Key (1 on, 0 off)

bit 4: Flashlight in Multmedia > Camera > Pictures (E398) (1 on, 0 off)

bit 5: Use Info Services Alert for Browser Msg (1 on, 0 off)

bit 6: SMS Subject (1 on, 0 off)

offset 7A -	bit 4: Vibe and Ring (E398/V551) (1 on, 0 off)

bit 7: Settings > Phone Status > Other Information > Operator (1 on, 0 off)

offset 7B -	bit 2: Personalize > Phone Status > Software Update (1 on, 0 off)

offset 7C -	bit 1: Display: WVIM Branding Support (1 on, 0 off)

bit 2: Show signal, time, battery status when display goes to blank (1 on, 0 off) (Settings > Initial Setup > Display Timeout must be 1 or 2 minutes)

offset 7D -	bit 5: Alert Ramp Up (Increasingly Louder Ringtone) (1 on, 0 off)

bit 7: Multimedia > Video > Menu > Videos Setup > Resolution (1 on, 0 off)

offset 7E -	bit 0: Handsfree immediately available when starting a call (1 on, 0 off)

bit 1: Multimedia > Videos > Videos Setup > Zoom (1 on, 0 off)

offset 7F -	bit 0: Message > Message Setup > MMS Message Setup, Size Limit (1 on, 0 off)

bit 1: Auto Network on Power Up (1 on, 0 off)

offset 80 - 	bit 2: Poll Headset for TTY Call (1 on, 0 off)

bit 6: Settings > Initial Setup > Backlight length options (1 for 20/40/60 seconds, 0 for 5/10/20 seconds)

offset 81 -	bit 2: Multimedia > Videos > Videos Setup > Styles > Black and White (1 on, 0 off) (offset 82, bit 0 must be set to 1)

bit 3: Multimedia > Videos > Videos Setup > Styles > Bluish (1 on, 0 off) (offset 82, bit 0 must be set to 1)

bit 4: Multimedia > Videos > Videos Setup > Styles > Antique (1 on, 0 off) (offset 82, bit 0 must be set to 1)

bit 5: Multimedia > Videos > Videos Setup > Styles > Reddish (1 on, 0 off) (offset 82, bit 0 must be set to 1)

bit 6: Multimedia > Videos > Videos Setup > Styles > Greenish (1 on, 0 off) (offset 82, bit 0 must be set to 1)

bit 7: Multimedia > Videos > Videos Setup > Styles > Negative (1 on, 0 off) (offset 82, bit 0 must be set to 1)

offset 82 -	bit 0: Multimedia > Videos > Videos Setup > Styles (1 on, 0 off)

bit 1: Multimedia > Video Camera > Brightness Adjustment (1 on, 0 off)

bit 2: Display: Allow Screensaver If J2ME Active (1 on, 0 off)

bit 6: Keynote Sound for Keypresses (1 on, 0 off)

bit 7: Multimedia > Videos > Videos Setup > Video Length (1 on, 0 off)

offset 83 -	bit 4: right softkey function after taking picture/video (1 SEND, 0 STORE)

bit 7: USB Settings (1 on, 0 off)

offset 84 -	bit 3: Multimedia > Pictures > Pictures Setup > Image Style (1 on, 0 off)

bit 4: Create Message > MMS > Priority > Low (1 on, 0 off)

bit 5: SIMTK Refresh: No Please Wait Notice (1 on, 0 off)

offset 85 -	bit 2: Multimedia > Camera > Camera Setup > Resolution (1 on, 0 off)

bit 4: Multimedia > Camera > Camera Setup > Auto Timer (1 on, 0 off)

bit 6: Airplane Mode Lock (1 on, 0 off) (menu item, may require update to mma files) (confirmed not to work on V551)

bit 7: Enhanced Fixed Dial (1 on, 0 off)

bit 7: CPHS Orange Available (1 on, 0 off)

offset 86 -	bit 1: Call Barring > BOAC Available (1 on, 0 off)

bit 2: Call Barring > BOIC Available (1 on, 0 off)

bit 3: Call Barring > BOIC_EXHC Available (1 on, 0 off)

bit 4: Call Barring > BAIC Available (1 on, 0 off)

bit 5: Call Barring > BIC Roam Available (1 on, 0 off)

bit 6: Call Barring > CLIP Available (1 on, 0 off)

bit 7: Line 2 Call Forward

offset 87 -	bit 0: Line 2 Conditional Call Forward (1 on, 0 off)

bit 1: Line 2 If Busy, then Call Forward (1 on, 0 off)

bit 2: Line 2 If No Answer, then Call Forward (1 on, 0 off)

bit 3: Line 2 If Not Reachable, then Call Forward (1 on, 0 off)

bit 4: Line 2 Call Barring (1 on, 0 off)

bit 5: Call Barring > BOAC Line 2 Available (1 on, 0 off)

bit 6: Call Barring > BOIC Line 2 Available (1 on, 0 off)

bit 7: Call Barring > BOIC_EXHC Line 2 Available (1 on, 0 off)

offset 88 -	bit 0: Call Barring > BAIC Line 2 Available (1 on, 0 off)

bit 1: Call Barring > BIC Roam Line 2 Available (1 on, 0 off)

bit 2: Line 2 Call Hold (1 on, 0 off)

bit 3: Line 2 Call Waiting (1 on, 0 off)

bit 4: MT SMS Line 2 (1 on, 0 off)

bit 5: MO SMS Line 2 (1 on, 0 off)

bit 6: CLIP Line 2 Available (1 on, 0 off)

bit 7: Allow CLI Send (Calling Line Identifier) Line 2 (1 on, 0 off)

offset 89 -	bit 0: Allow CLI Block (Calling Line Identifier) Line 2 (1 on, 0 off)

bit 1: Voicemail Line 2 Available (1 on, 0 off)

bit 2: EDGE Icon (1 on, 0 off) (only on EDGE supported phones with late version firmware)

bit 5: Swap Softkeys (1 on, 0 off)

bit 6: Multimedia > Camera > Camera Setup > Image Quality (1 on, 0 off)

offset 8A -	bit 1: Multimedia > Videos > New Video > Pause/Resume Capture (1 on, 0 off)

bit 2: Speaker Call to End on Flip Close (1 on, 0 off)

offset 8B -	bit 1: Allow DRM File Download To Ext Storage (1 on, 0 off)

bit 2: Multimedia Video Player Stretch Video (1 on, 0 off)

bit 4: Cingular ENS Capable (1 on, 0 off)

bit 7: Multiple Skin Support (1 on, 0 off)

offset 8C -	bit 0: Multimedia: Store> Store Options > Send Postcard

bit 2: Allow Use of Pictures on Removable Media for Phonebook (1 on, 0 off)

offset 8D -	bit 0: USSD Alert Available (1 on, 0 off)

bit 4: UMAN Feature Enabled (1 on, 0 off)

offset 93 -	bit 0: V3i: Settings > Phone Status (1 on, 0 off)

bit 1: V3i: Settings > Personalize (1 on, 0 off)

bit 2: V3i: Settings > Network (1 on, 0 off)

bit 3: V3i: Settings > Ring Styles (1 on, 0 off)

bit 4: V3i: Settings > Security (1 on, 0 off)

bit 5: V3i: Settings > Initial Setup (1 on, 0 off)

bit 7: V3i: Messages, blue Message button (1 on, 0 off)

offset 94 -	bit 0: V3i: Settings > Car Settings (1 on, 0 off)

bit 1: V3i: Settings > Headset (1 on, 0 off)

bit 3: V3i: Settings > In-Call Setup (1 on, 0 off)

bit 4: V3i: Phonebook (1 on, 0 off)

bit 5: V3i: Tools > Shortcuts (1 on, 0 off)

offset 96 -	bit 4: V3i: Camera Orientation (1 normal, 0 flipped horizontal and vertical)

offset 9A - 	bit 1: Messages > menu > Message Setup > Text Msg Setup > Store To: (SIM/Phone) (1 on, 0 off)

bit 7: BLOCK Multimedia > Sounds ability to create playlists and hide player (1 on, 0 off) (set to 0 to allow playlists and hiding of player)

offset AE -	bit 0: V3i: Camera IU (1 transparent, 0 opaque/V3 style)

========

spcolor.sch

upload to /a/mobile/system/

activate bit #6 of offset 53 in seem 0032_0001

**thanks to poesjeNL and FlyingHorse for the English translation**

034 - background color messages

038 - background color main menu

03C - color of characters in menu and the inner color of the characters on the main screen

040 - color of the text of a selected item

044 - color of the text at the discription of the pictures/ringtones/video

048 - ???

04C - textcolor of calendar entry for "today"

050 - ???

054 - color of hour- and minute-hands on analog clock

058 - border color the text on the main screen

05C - textcolor of the bottom bar

060 - textcolor at button press

064 - textcolor titlebar (main menu)

068 - textcolor titlebar (submenu/input area)

06C - upper color selection bar

070 - bottom color selection bar

074 - color line above bottom bar and right of scroll-bar

078 - upper color bottom bar

07C - bottom color bottom bar

080 - upper color of pressed button

084 - bottom color of pressed button

088 - upper color titlebar (above main menu)

08C - bottom color titlebar (above main menu)

090 - upper color titlebar (above submenu and input)

094 - bottom color titlebar (above submenu and input)

098 - color right line of horizontal bar at files (pictures/ringtones/filmpjes)

09C - background color of iTap line, symbols that are not active

0A0 - background color of calculator

0A4 - background color input/selection area

0A8 - background color of the information at multimedia (pictures/ringtones/video)

0AC - color of line under the input area

0B0 - color of line under the input area when selected

0B4 - color of cursor at input

0B8 - ???

0BC - horizontal line in calendar

0C0 - vertical line in calendar

0C4 - main color in calendar (weeks/months)

0C8 - background color of calendar entry for "today"

0CC - horizontal solution line in calculator (between 2+2 and 4)

0D0 - "emergency call" upper color titlebar

0D4 - "emergency call" bottom color titlebar

0D8 - "emergency call" color of line surrounding input area and titlebar

0DC - color of line surrounding icons

0E0 - color of line surrounding upper side of input area

0E4 - color of line surrounding bottom side of input area

0E8 - color of line surrounding submenu's and left and right sude of input area

0EC - color 1e line of scroll-bar

0F0 - color 2e line of scroll-bar

0F4 - background color of scroll-bar

0F8 - color bottom of scroll-bar

0FC - color of upper/left side of scroll block when playing a video

100 - upper background color of the bar at multimedia (under pictures/ringtones/videos/camera)

104 - bottom background color of the bar at multimedia (under pictures/ringtones/videos/camera)

108 - upper color of scroll-bar

10C - ???

110 - ???

114 - ???

118 - background color of a selected main menu item

11C - background color of the text at the buttond/options under the camera

120 - color for links in sound/pictures menu (certain branded flexes only)

124 - ???

128 - color for links in sound/pictures menu when highlighted (certain branded flexes only)

12C - ???

Specify colors in spcolor.sch in hexadecimal RGB.

The offset specified is the starting point - each six digit color is actually stored in three successive offsets.

For example: offset 78, upper color bottom bar: if you want the color FF1432, put "FF" in offset 78, "14" in offset 79, and "32" in offset 7A.

Certain fields can have transparency - that is the fourth successive offset, specify transparency amount from 00 to FF.  This will not work for all color settings.