This software is for upgrade of firmware for Motorola HT820 stereo Bluetooth headphone. Please unzip the whole package to a directory.

The screen capture I think is from Win 2000, so Win XP may look a bit different. The directory in the screen capture may also be different from your setup.

STEPS：

- Check that part number of your headphone should be “SYN0948A”、“SYN0948B”、 “SYN0948C”

- Requirements :

  - PC installed with Windows 2000 or XP

  - USB to mini USB cable

- Connect HT820 with PC using USB to mini USB cable

  - Press“forward －>”and “backward <－”buttons together

  - Plug mini USB into HT820

  - Wait 3 seconds

  - release“forward －>”和 “backward <－”butoons

  - LCD blue light on HT820 should be flashing

    - If light does not flash, charge HT820 for a few minutes and retry

    - If light is flashing, install driver as below

- Install CSR USB driver

- Run “DFU_flash_tool” DFU wizard (DFUWizard.exe)

- Depends on whether your model is SYN0948A, B or C :-

      - if SYN0948A, install neptuneRevA_v0116.dfu

      - if SYN0948B or SYN0948C, install neptuneAM_v0116.dfu

- Installation takes about 90 seconds

Choose Next, takes about 90 seconds

Wait for 30 seconds for processing data

- Final message – Successful Upgrade

    - If “Upgrade Failed”,  recharge headphone for 3 minutes，repeat step 3

- remove mini USB cable

- Done!!!