*2009 UPDATE ARGON SEEM MAP*

NEW SEEM MAP FOR THE P2K05 PHONES
Originaly created by xlr8, but since he stopped that work, allot of people was finding new seem edits in different forums, so I decided to put them all together in one list, all discovered seem map hacks are here, also added a few more new. There was a nice seem map made by StarkRaving at MMUS, big thanks to him for started the job. 
based in the same old way by xlr8, I've structured this new seem map.

Thanks to everyone who contribuited with this new seem map

- russoeternal
- OCM770
- kent_lkc
- Malta11 
- Semseddin 
- StarkRaving
- RaMi
- daywalker04
- Skrilax_CZ
- xlr8
- surgicalknot
- bobby3
- omygodisitme
- poetic_folly

========

DISCLAIMER

All these seem edits were tested by allot of users in forums, use them at your own risk, remember, this hacks will modify your Motorola phone.

If you found a new seem edit and it's not listed in here, please contact to russoeternal via PM on the following forums: www.modmymoto.com | www.motoxtreme.info | www.motoevolution.net | www.mmus.us | www.smg.skinnermania.com | www.hellomotorazrforum.com | www.motomodtr.forumcommunity.net |

Use the search if don't see what you need.

DO NOT POST THIS LINK WITHOUT RUSSOETERNAL'S PERMISSION.

========

Начало формы

Конец формы

Custom Search

========

P2K05 PHONE LIST

- RAZR V6/RAZR MAXX/MAXX V6
- RAZR V3XX/MAXX V3/V3XXR
- KRZR K3/MAXX K3
- RAZR2 V9
- RIZR Z9

========

Other Interesting Seems:

0016 - Stores Hardware Version
0037,1-3 - Quick Dials 1,2,3
005B - Key Mapping
0061 - CL.gif on phone
0062 - CL.gif on phone
0074 - Stores Phone Lock Code
0076 - Stores Phone Security Code
0117 - Phone Model Number
0132 - Bluetooth Name
013A - Greeting Text
0149 - Quick Dials Name
0171 - Other Information > Technology
017F - Flex Version
022D - Screensaver location
0237 - Wallpaper image
0238 - Wallpaper image
02D2 - Firmware Version

========

SEEM 00a2_0001

**all offsets are HEX**

offset 00 - bit 5: disable AMR Half Rate (1 on, 0 off) (bit 6 must be off)
bit 6: enable AMR Half Rate (1 on, 0 off) (bit 5 must be off)

offset 01 - bit 5: disable AMR Full Rate (1 on, 0 off) (bit 6 must be off)
bit 6: enable AMR Full Rate (1 on, 0 off) (bit 5 must be off)

How to tell what audio codec your phone is using:
While in a call, enter field test mode and in the Serving CS/TB screen, look on the bottom line beside "coder."
The first part of that string of letters is the part you're looking for.
AFS = AMR Full Rate
AH0 = AMR Half Rate
AH1 = AMR Half Rate
EFR = Enhanced Full Rate
FR = Full Rate

========

SEEM 004a_0001/004b_0001

**all offsets are HEX**
**no bit manipulation here, just change the actual value in the offset**

**SOME PEOPLE HAVE NOTED MIXED RESULTS WITH THESE HACKS; THEY MAY OR MAY NOT WORK FOR YOU**

offset 22 (01 for custom startup sound stored in /a/mobile/audio/Start~up.mp3, 00 for no custom startup sound), may need to adjust 0032_0001, offset 24, bit 3
offset 23 (01 for custom startup in stored seems 0206/0208, 00 for no custom startup)
offset 24 (01 for custom shutdown sound stored in /a/mobile/audio/Power~down.mp3, 00 for no custom shutdown sound)
offset 27 (00 for custom outer image from path saved in seems 0061/0062, 01 for custom outer text stored in seems 0061/0062, 02 for Motorola icon)
offset 4D (00 SIM card NOT required for phone access, 01 SIM card required for phone access)
offset 80 (01 for number display with no dashes, 00 for number display with dashes- 1-404-555-1212)
offset 111 (01 for MTWTFSS week format in calendar, 00 for SMTWTFS week format in calendar)
offset 1A1 (01 for no WMV support, FF for WMV support) see also offset 0310)
offset 1BE (00 for no deletion of inbox, outbox, drafts messages on sim swap)
offset 1D1 (01 for .3gp video record, 03 for .mp4 video record)
offset 1ED (swap to ring style when press/hold # key (01 soft, 02 vibrate, 03 vibe then ring, 04 silent, 05 vibe&ring)
offset 207 (01 charge via computer USB connection, 00 do not charge via computer USB connection)
offset 310 (01 for no WMV support, 00 for WMV support) see also offset 01A1

**all offsets are HEX**

offset 10F - bit 0: Open Multimedia menu with browser key (0 on, 1 off)
- bit 1: Open Multimedia menu with browser key (1 on, 0 off)

========

SEEM 004e_0001

**all offsets are HEX**
**no bit manipulation here, just change the actual value in the offset**
**SOME PEOPLE HAVE NOTED MIXED RESULTS WITH THESE HACKS; THEY MAY OR MAY NOT WORK FOR YOU**

offset 338, 339, 33A (00 for video record unlimited) for old firmwares.
offset 48B (64 for improve the camera quality) see also offset 4CB
offset 4CB (09 for improve the camera quality) see also offset 48B
offset 490, 491, 492, 493, 494, 495, 496, 497, 498, 499, 49A, 49C, 49D, 49E, 49F (00 for improve VideoCamera quality)
offset 49B (18 for improve VideoCamera quality)
offset 4A0, 4A1, 4A2, 4A4, 4A5, 4A6, 4A7, 4A8, 4A9, 4AA, 4AB, 4AC, 4AE (00 for improve VideoCamera quality)
offset 4A3 (30 for improve VideoCamera quality)
offset 4AF (10 for improve VideoCamera quality)
offset 4B3 (03 for improve VideoCamera quality)

========

SEEM 0126_0001

**all offsets are HEX**

offset 03 - bit 5: Items Displayed in Recent Calls > Dialed Calls (1 to display only most recent call to same number, 0 to display all calls to the same number)
bit 6: Items Displayed in Recent Calls > Received Calls (1 to display only most recent call from same number, 0 to display all calls from the same number)

========

SEEM 0032_0001

bits: 00000000 <-- binary
76543210 <-- bit #

**all offsets are HEX**

offset 00 - bit 0: Initial setup > Time and Date > Autoupdate (1on, 0 off)
bit 2: Settings > Initial Setup > Auto Redial (1 on, 0 off)

offset 02 - bit 1: BLOCKS THESE FEATURES: Call barring, Call transfer, Info Services, do not read VM number from SIM, Voicemail ^@ text messages, Multi-Line Support, My Network List, Manual Network Search (1 on, 0 off - set to 0 to UNBLOCK these features) 
bit 1: Operator carrier from plmn_text_table.bin (0 on, 1 off) See offsets 4B and 40 also.
some of these features may be SIM dependant anyway, so activating it on your phone may not actually allow you to use it
bit 4: Test mode engineering menu, require menu edit (1 on, 0 off)

offset 03 - bit 0: General broadcasting (1 on, 0 off)
bit 1: Message > Quick Notes (1 on, 0 off)
bit 2: Message > Drafts (1 on, 0 off)
bit 3: Message > Outbox (1 on, 0 off)
bit 5: Message > Inbox (1 on, 0 off)

offset 04 - bit 1: Call Restriction Support (1 on, 0 off)

offset 08 - bit 5: Recent Calls > Call Cost (1 on, 0 off)

offset 09 - bit 0: Settings > Phone status > Voicecommands (1 on, 0 off)
bit 1: Settings > Security > Talk Secure (1 on, 0 off)

offset 0A - bit 7: Settings > Initial Setup > Backlight > Continuous (1 on, 0 off)

offset 0F - bit 0: Text "Additional cost may be applied" when you open the browser

offset 14 - bit 2: Quick Dial (1 on, 0 off)
bit 5: Settings > Network > New Network (1 on, 0 off)

offset 15 - bit 3: AGPS Service (1 on, 0 off)

offset 16 - bit 6: Settings > Ring Styles > Details > Roam Calls option, special ring tone for calls received while roaming (1 on, 0 off)

offset 1B - bit 6: Settings > Initial Setup > Auto PIN Dial (1 on, 0 off)

offset 1C - bit 0: Settings > Initial Setup > Preferred SIM Card (1 on, 0 off)

offset 1D - bit 3: Browser slash screen (1 from DRM, 0 from mib_op_greet)
bit 6: Settings > Initial Setup > Battery Save (1 on, 0 off)

offset 1E - bit 3: Settings > Initial Setup > Animation (1 on, 0 off) I HAVE TO TRY IT
bit 7: Settings > In-Call Setup > Talk and Fax > All Calls (1 on, 0 off)

offset 1F - bit 4: 1-2 Line, it shows the Line 1 and Line 2 at the top of the screen (1 on, 0 off)
bit 7: Fax Calls (1 on, 0 off)

offset 20 - bit 1: Talk and Fax - Next call only (1 on, 0 off)

offset 22 - bit 6: Settings > Call Divert/Forward > Fax (1 on, 0 off)
bit 7: Settings > Call Forward > Video Calls (1 on, 0 off)

offset 23 - bit 5: Call Guard Option [confirmation for outgoing calls when roaming] (1 on, 0 off)

offset 24 - bit 2: Unlock Quick Dial 1 (0 on, 1 off)

offset 29 - bit 1: Lock keypad (1 on, 0 off) Just works with RAZR V3X

offset 2F - bit 2: Webaccess > Web Sessions > Edit > IP1 (1 on, 0 off)
bit 3: Webaccess > Web Sessions > Edit > IP2 (1 on, 0 off)
bit 4: Webaccess > Web Sessions > Edit > CSD1 (1 on, 0 off)
bit 5: Webaccess > Web Sessions > Edit > CSD2 (1 on, 0 off)

offset 34 - bit 3: Roam Indicator Triange [See offset CB bit 5] (1 on, 0 off)

offset 37 - bit 6: Network Settings > USB Settings > Network monitor (1 on, 0 off)

offset 3F - bit 4: Disable unicode (1 on, 0 off)

offset 3D - bit 1: SMS Delivery Report [with just the others will be for MMS] (1 on, 0 off)
bit 3: Countdown the characters on SMS (1 on, 0 off)

offset 40 - bit 1: GPRS icon display
bit 4: Operator carrier from plmn_text_table.bin (0 on, 1 off) See offsets 4B and 02 also.

offset 41 - bit 5: GPRS Active Icon (1 on, 0 off)

offset 43 - bit 0: Java App Loader (1 on, 0 off)
bit 1: Delete all apps from phone, even the locked ones (1 on, 0 off)
bit 4: Network > Network Setup > Band (1 on, 0 off)

offset 44 - bit 4,5,6,7: Testmode engineering menu, will require mma edit (1 on, 0 off)

offset 48 - bit 4: Settings > Call Divert/Forward > Voice Calls > Conditional (1 on, 0 off)
bit 5: Settings > Call Divert/Forward > Voice Calls > Busy (1 on, 0 off)

offset 49 - bit 5: AGPS icon on top (1 on, 0 off)

offset 4A - bit 0: Personalize > Home Screen > Banner (1 on, 0 off)

offset 4B - bit 6: Operator carrier from plmn_text_table.bin (0 on, 1 off) See offsets 40 and 02 also.

offset 4C - bit 2: Phone status > Other information > MP3 info (1 on, 0 off)

offset 4D - bit 1: Initial setup > TTY Setup (1 on, 0 off)

offset 4F - bit 6: Messaging > E-Mail Msgs (1 on, 0 off)

offset 50 - bit 3: "Emergency Only" display when network available but no roaming agreement (1 on, 0 off)
bit 4: "No Service" display when you have no service (1 on, 0 off)

offset 51 - bit 3: Alpha Tag Display for V9/Z9 (0 from phone, 1 from SIM)
bit 7: WCDMA 2100 shown in Network > Network Setup > Band (1 on, 0 off)

offset 53 - bit 1: Zoom on external display (1 on, 0 off)
bit 2: Initial setup > Contrast (1 on, 0 off) apparently doesn't change anything

offset 57 - bit 6: 900/1800 shows in Network > Network Setup > Band (1 on, 0 off)

offset 58 - bit 1,3,4,5: Shutdown Sound (1 on, 0 off)

offset 5A - bit 0: Dynamic prompts (1 on, 0 off) I HAVE TO TEST

offset 5B - bit 0: Internal Camera and Video (1 on, 0 off)
bit 4: Address Book > Contact > Video Number (1 on, 0 off)

offset 5C - bit 0: Music/Videos Setup > Timer (1 on, 0 off)
bit 2: Video camera > Settings > Remind (1 on, 0 off)

offset 5D - bit 0: Camera & Video Setup > Mirror (1 on, 0 off)

offset 5E - bit 1: Messaging > Templates > Multimedia Templates (1 on, 0 off)

offset 60 - bit 0: Alarm Clock (1 on, 0 off)
bit 6: Speed Dial #2 (unknown on/off)

offset 61 - bit 3: Confirmation to delete (1 on, 0 off)

offset 62 - bit 4: SMS over GPRS Available (1 on, 0 off)
bit 6: 850/1900 shows in Network > Network Setup > Band (1 on, 0 off)
bit 7: 1800 shows in Network > Network Setup > Band (1 on, 0 off)

offset 63 - bit 0: WCDMA 1900 (1 on, 0 off)
bit 1: European band (1 on, 0 off)
bit 2: North America band (1 on, 0 off)

offset 66 - bit 5: VM message counter in Messages > Voicemail (0 on, 1 off)

offset 69 - bit 2: Initial setup > Video gateway interactive setup (1 on, 0 off)

offset 6B - bit 2: Priority settings for Java apps (1 on, 0 off)
bit 3: Java Switch to Debug (1 on, 0 off)
bit 4: Start TCK (1 on, 0 off)
bit 5: Show Java splash screen (1 on, 0 off)

offset 6C - bit 1: MMS Size limit (1 on, 0 off)

offset 70 - bit 7: for Thai language and iTap support (1 on, 0 off) just for Asian firmwares

offset 72 - bit 1: parse e-mail gateway to show actual sender's e-mail for e-mail received as text (1 on, 0 off)

offset 73 - bit 0: Text Message Reply > Options > Change Recipients (1 on, 0 off)
bit 6: FW: on SMS when it's forwarded (1 on, 0 off)
bit 7: RW: on SMS when it's replied (1 on, 0 off)

offset 74 - bit 4: Settings > Audio > Ring Lights (1 on, 0 off)

offset 75 - bit 1: IM (1 on, 0 off)

offset 78 - bit 3: Camera > Options > Light On (1 on, 0 off)
bit 5: Games and Applications - settings protection (1 on, 0 off)

offset 79 - bit 1: Recording incoming videocalls (1 on, 0 off)

offset 7E - bit 2: Network jingle (1 on, 0 off)

offset 7F - bit 1: Get an extra bar of signal (1 on, 0 off)
bit 6: Swap softkeys (1 on, 0 off)

offset 80 - bit 6: Phone status > Software update (1 on, 0 off)

offset 82 - bit 3: Security > Call barring (1 on, 0 off)

offset 83 - bit 1: Light time (1 for 20-40-60, 0 for 5-10-20)

offset 87 - bit 1: Display status of camera LED when taking a picture (1 on, 0 off)

offset 89 - bit 6: Prepend Re: and Fw: to subject on text message replies and forwards (1 on, 0 off)

offset 8A - bit 1: Pause the video on both cameras by pressing the central button (1 on, 0 off)

offset 8B - bit 2: allow speakerphone with flip closed (0 on, 1 off) (microphone will not work)

offset 8D - bit 4: Wakeup Bootscreen (1 on, 0 off)
bit 6: Type of Wakeup Bootscreen (1 motorola, 0 provider) See the seem 033F for location
bit 7: Type of Wakeup External Bootscreen (1 motorola, 0 provider) See seem 0340 for location

offset 90 - bit 6: Change to Loud Ring Style when charging or connected by USB (0 on, 1 off)

offset 9B - bit 6: Personalize > Voice commands (1 on, 0 off)

offset 9E - bit 0: EMS support in text messages (0 on, 1 off)

offset 9F - bit 3: IM listed in Message menu (1 on, 0 off) (IM must be turned on, see offset 75, bit 1)
bit 7: Japan band listed on Bands (1 on, 0 off)

offset A0 - bit 0: WCDMA 850/1900 Band (1 on, 0 off)

offset A1 - bit 5: GSM/GPRS then UMTS band (1 on, 0 off)
bit 6: UMTS then GSM/GPRS band (1 on, 0 off)

offset A6 - bit 0: Ring delay (1 on, 0 offset)

offset A8 - bit 1,2,3,4: Testmode engineering menu, will require mma edit (1 on, 0 off)

offset A9 - bit 5: Basic Settings (1 on, 0 off)

offset AE - bit 4: Full view on internal camera screen (1 on, 0 off)

offset B1 - bit 0: Messaging > Create Message > Prompt for Message Type (ie. SMS/MMS, Voice, E-Mail) (1 on, 0 off)
bit 3: BLOCK ADVANCED MENUS: Settings > Network (1 on, 0 off - set to 0 to UNBLOCK advanced Network Menus)

offset B2 - bit 0: Settings > Audio > Event Lights (1 on, 0 off)
bit 4: Disable EDGE Support (0 on, 1 off)

offset B4 - bit 5: Settings > Phone Status > Software Update > Check Server (1 on, 0 off)

offset BB - bit 1: Personalize > Homescreen > SCREEN3 (1 on, 0 off) it will require Launchbar
bit 6: Personalize > Titles (1 on, 0 off) it will require Launchbar

offset BE - bit 1: SMS Delivery report (1 on, 0 off) see offset 9E

offset C9 - bit 0: Enable the "Hot messages" folder on Message centre (1 on, 0 off)
bit 5: "Searching..." display when phone is searching for network (1 on, 0 off)

offset CA - bit 1: Dialed, received, missed calls all in one list (1 on, 0 off)

offset CB - bit 0: PTT Enabled (1 on, 0 off) will require mma_ucp edit
bit 5: "generation" indicator, 3G/2.5G/etc (this may be the roam status indicator... it's in that spot) (0 on, 1 off)

offset CC - bit 0: Messaging > MMS Settings > Server info (1 on, 0 off)
bit 1: Messaging > MMS Settings > Server info > Name (1 on, 0 off)
bit 2: Messaging > MMS Settings > Server info > Address (1 on, 0 off)
bit 3: Messaging > MMS Settings > Server info > Websession (1 on, 0 off)

offset CE - bit 6: Settings > Call Forward > Push to Call Forward Number

offset CF - bit 3: show "3G" instead of GPRS indicator when in 3G area (1 on, 0 off)

offset D0 - bit 0: HSDPA Icon (shows only when in internal data session) (1 on, 0 off)
bit 1: show HSDPA data transfer icon (1 on, 0 off)
bit 2: HSDPA indicator and transfer icons for external (tethered) data sessions (1 on, 0 off)

offset D1 - bit 3: MMS Send priority (0 on, 1 off)
bit 5: force list view when more than 6 items in a submenu (1 icons, 0 list)

I'll be updating this page as soon as we find more new seem edits. russoeternal, click on ads to support.