Краткая инсрукция DMTool:

Краткая инсрукция DMTool:

COM-кабелем

Прошивка С261 - DM Tool 1.2.0.9, всё ложить в корень диска С:

Зарядите аккумулятор на 100%.

Подключаете кабель.

Выключаете телефон.

Порт: Выбираем куда подключён

Cкорость: Normal

Лоадер: E58S_MP(на w-серии надо выбрать К2 или Key 2)

Включить: Проверка АКБ и Блоков

Последовательность в списке (Можно шить по 1 файлу. Прошивка и ленг могут не совпадать по версии):

Прошивка: V0.1.66_Secure_MP_Key2.mot

Ленгпак: lang_pack_12.mot

флекс: FLEX_90_61_1203.mot

» Start

В этот момент Вы включаете телефон. Если через 10 секунд Вы его не включили программа выдаст cooбщение. USB-кабель

Прошивка С261 - DM Tool 1.2.0.9, всё ложить в корень диска С:

Зарядите аккумулятор на 100%.

Подключаете кабель.

Выключаете телефон.

Порт: USB-COM4 или тот на котором подвешен телефон.

Cкорость: USB-Ultra

Лоадер: E58S_MP (на w-серии надо выбрать К2 или Key 2)

Включить: Проверка АКБ и Блоков

Последовательность в списке (Можно шить по 1 файлу. Прошивка и ленг могут не совпадать по версии):

Прошивка: V0.1.66_Secure_MP_Key2.mot

Ленгпак: lang_pack_12.mot

флекс: FLEX_90_61_1203.mot

» Start

В этот момент Вы включаете телефон. Если через 10 секунд Вы его не включили программа выдаст cooбщение об ошибке.

Перед заменой ПО, проверьте текущую версию в тел., набрав в режиме ожидания #02#, понижать версию не стоит. Но если очень надо - сделать следующее:

найти в папке дмтула файл \"fldevice_XXX.ini\

удалить в файле строки:

[VER_STAMP_INFO]

VerHwIdAddr 	=ххххххххх

VerSwIdAddr 	=хххххххх

TimeStampIdAddr=ххххххх

[VER_MOT_INFO]

VerFlexAddr	= ххххххх

VerLnPkAddr	= хххххх

VerRsPkAddr	= хххххх (находится в папке с DMTool)

После этой операции, DMTool должен согласиться прошить старую версию прошивки

Перед прошивкой рекомендую сделать backup.

( на примереW218)

Открываем

Выбираем com port – speed  115200- Connect.

Выбираем Eeprom (самое важное) – нажимаем Read.  Проделываем тоже самое с FullFlash.

С программой

работаем также. Она используется для моделей C261,C257,V176,V177,W37x.

Удачной прошивки!!!!

P.S.  Полезная информация:

V175V176V177C257C261 Language packs description V175V176V177C257C261 Language packs description
V175/V176/V177/C257/C261 EMEA 16 Language Bundle 

Language pack 1: UK English, French, German, Italian 
Language pack 2: UK English, French, German, Dutch 
Language pack 3: UK English, Spanish, Portuguese, Greek 
Language pack 4: UK English, Norwegian, Danish, Swedish 
Language pack 5: UK English, Swedish, Finnish, Estonian (*) 
Language pack 6: UK English, Lithuanian (*), Latvian (*), Russian 
Language pack 7: UK English, Serbian (*), Slovenian (*), Croatian (*) 
Language pack 8: UK English, Hungarian, Polish, Czech, Slovak 
Language pack 9: UK English, Russian, Arabic (*), Hebrew (*) 
Language pack 10: UK English, German, Greek, Russian, Arabic (*), Hebrew (*) 
Language pack 11: UK English, French, Arabic, Urdu, Farsi 
Language pack 12: UK English, German, Russian, Ukranian (*) 
Language pack 13: UK English, French, German, Dutch (*), Portuguese, 
Language pack 14: UK English, German, Italian, Russian (*), Turkish 
Language pack 15: UK English, German, Dutch, Turkish 
Language pack 16: UK English, Albania, Macedonia 

Note: languages marked with (*) has only TAP support (doesn't have iTAP) 

__________________________________________________ _________
W205W208 Language packs description
HGM: 

Language pack H01: Telugu, Hindi, English_UK, Bengali, Tamil 
Language pack H02: Bahasa_indonesia, Vietnamese, Thai, English_UK 
Language pack H03: Vietnamese, Greek, Chinese_Simplified, English_UK, Italian 
Language pack H04: English_UK, Hebrew, Russian, Arabic 
Language pack H05: Farsi, English_UK, Urdu, Arabic, French 
Language pack H06: French, Portuguese, English_UK, Dutch, German 
Language pack H07: Italian, Russian, English_UK, German, Turkish 
Language pack H08: English_UK, Yoruba, Portuguese, French, Hausa, Sesotho, Afrikaans, Ibo, Swahili, Zulu 
Language pack H09: Bahasa_Malay, English_UK, Chinese_Simplified 

__________________________________________
W375 Language packs description

W375 EMEA 17 Language Bundle 

Language pack 1: UK English, French, German, Italian 
Language pack 2: UK English, French, German, Dutch 
Language pack 3: UK English, Spanish, Portuguese, Greek 
Language pack 4: UK English, Norwegian, Danish, Swedish 
Language pack 5: UK English, Swedish, Finnish, Estonian (*) 
Language pack 6: UK English, Lithuanian (*), Latvian (*), Russian 
Language pack 7: UK English, Serbian (*), Slovenian (*), Croatian (*) 
Language pack 8: UK English, Hungarian, Polish, Czech, Slovak 
Language pack 9: UK English, Russian, Arabic (*), Hebrew (*) 
Language pack 10: UK English, German, Greek, Russian, Arabic (*), Hebrew (*) 
Language pack 11: UK English, French, Arabic, Urdu, Farsi 
Language pack 12: UK English, German, Russian, Ukranian (*) 
Language pack 13: UK English, French, German, Dutch (*), Portuguese, 
Language pack 14: UK English, German, Italian, Russian (*), Turkish 
Language pack 15: UK English, German, Dutch, Turkish 
Language pack 16: UK English, Albania, Macedonia 
Language pack 17: UK English, Lithuanian (*), Latvian (*), Russian, Polish 

Коды:
Set language to English: *#0044# 
Set language to Russian: *#0007# 
Service codes Motorola: 
Motofone F3 software version: **9999* Send 
C113, C114, C115, C115i, C116, C117, C118 software version: #02#* 
C138, C139, C140 software version: #02#* 
C155, C156, C157 software version: #02#* 
C257, C261 software version: #02#* 
V171, V172, V173 software version: #02#* 
V175, V176, V176 software version: #02#* 
C168, W220 software version: *#**837# 
W208, W375 software version: #02#*

OSA86